MD730C64 is a version of Irv Hoff's MDM730 modified for

the Commodore 64.  If you are unfamiliar with MDM730, you can

get a detailed description from the file MDM730.DQC in the

library MDM730.LBR.  This version works with the Hayes 300

Smartmodem, the VICmodem and probably any other modem that

connects to an RS-232 port.  With the Hayes, you can use the

auto dial features of the program.

MD730C64 will work ONLY with a 44k (or smaller) Commodore

64 CP/M system.  The RS-232 buffers and a character translation

table are located in the "extra" 4k.  If you try to work with

a 48k system, the program will crash.

Please note the following:

1. Ctl-W (not Ctl-T) is used to start an ASCII file transfer

in terminal mode.

2. The program is currently set for pulse dialing with a

Haye's Smartmodem.  If you have touch tone dialing, patch

location 105H to be an ASCII 'P'.  It is currently a 'T'.

3. There is no cursor when you are in terminal mode.  (You'll

get used to it - honest!)

4. The program has been tested at 300 baud.  Because of

the interaction between the two processors, it may need

more work to run correctly at 1200.

This is an initial release and I would sincerely appreciate

hearing from you if you use the program.  Modifying MDM730 for

this computer was painful because of the way the C-64 handles its

RS-232 port.  It has been thoroughly tested but it is

a very complex program and there may be problems remaining simply

because there are combinations of functions that I am not aware of.

Please let me know if you incover any.  I'd also like to hear from

you if you use it and don't have problems.  When I have enough

confidence in the program,  I'll release the source code for

the overlay.

Walt Piotrowski

RD1 Box 582

Afton, NY 13730

(607) 693-2809 (voice)

Compuserve 72636,761

or

Leave a message on Charles Zalenski's

RBBS (607) 797-6416 - Johnson City, NY