Date: Thursday, 27 June 1985

From: Larry J. Huntley

Re:   One-of-a-Kind Systems (LONG!  Ph. Nos., Addresses)

I tried to reach a lot of the people who contacted me by mail, but

without a lot of success, so I will now subject the net to this

volume of verbiage.  I know this is long and some of it is redundant,

but I'm trying to answer a lot of different questions which I got as

a result of my previous postings.  Have patience.

The cut-strip-wrap tools we have been using here are manufactured by

Swiss Precision, Inc in Palo Alto.  The representative goes by the

name of OEM Associates in Sunnyvale.

The one we're currently using is for 30-ga wire, and makes a modified

wrap (the first 1-2 turns of the wire have insulation on them.) It's

called "CSMW", part no. 23430.

I recommend you contact OEM (408/374-0381) so that they can direct

you to the representative or distributor in your area.  These things

are listed in the current catalogs (Contact East for one) but at

outrageous prices ($221.65.)  The last ones I bought came from

Coastal Marketing in Canoga Park; they were around $185.

There is one caveat -- you need to be careful concerning the wire

that you use with these bits.  If the insulation adheres to the wire

too well, you will get wraps where the wire is not completely

stripped, i.e. small strands of insulation will be wrapped around the

post along with the conductor.  Regular Alpha Wire 30-ga. Kynar

insulated wire-wrap wire seems to be okay.  There is a special wire

made for these bits called Ad-Wrap; it's made by (are you ready?)

Advance Wire Co. As Howard Hull noted, you do have to be careful as

to what happens to the ends of the wire that the bit cuts off.  Most

of the time it falls into the forest of pins and wires on the wiring

side of the board.  Sometimes it's just insulation, sometimes it's

bare wire. Turn the board over and tap these guys out every once in

while and be sure to visually inspect the board after wrapping.  Of

course, you do that anyway, right?

Now, on to back-force guns.  I don't know if these can be retro-

fitted onto existing guns, and if it can, if it's a factory

modification or not.  Since I found out about them, I have always

bought tools with the back-force nosepiece fitted.  I would call the

manufacturer of your particular gun and ask.  Once again, if you're

buying new guns, expect to pay about $10 additional for these

widgets.

Here are some phone numbers; I hope they are of use.

Swiss Precision Inc.            Gardner-Denver

415/493-0440                    616/842-0200

Manufacturer of the             Wrapping tools and Equipment

"Cut, strip and wrap" bit.

OEM Associates                  OK Machine & Tool

408/374-0381                    212/994-6600

Rep. for Above                  More Tools

Coastal Marketing               Standard Pneumatic

714/241-7112                    702/329-6311

Distributor for Hand tools      More w/w tools

and production Equipment

Advance Wire & Cable            Alpha Wire & Cable

415/592-4550                    201/925-8000

Makers of "Ad-Wrap" wire        Makers of wire

Contact East

617/272-5051

Seller of Tools and Equipment

(Some basic comments on wiring techniques follow)

Wire-Wrap:

The components are interconnected with (usually) 30 gauge or

(sometimes) 26 gauge silver-plated annealed solid copper wire.  The

wire is insulated with one of several plastics, usually Kynar.  The

insulation is stripped off each end of the wire and the bare

conductor is wrapped around a tin-or gold-plated socket/pin (post.)

The post is 0.025" square and is usually die-cut such that the

corners are VERY sharp and they "bite" into the conductor as it wraps

around the post.  This results in a phenomenon called "cold-flow"

where the wire slowly comes into more intimate contact with the post

with time.  Very reliable.  The average wrapped connection includes

8 - 10 wraps around the post, so you get about 40 gas-tight

connections on each pin.  The Military and computer industry use the

method extensively.

Wire-wrapping wire can be bought in bulk (on spools) or in pre-cut,

pre-stripped lengths ranging from 1" to 2 feet or so.  There are

completely manual tools (you cut, you strip, you spin the tool to

make the wrap), battery/A.C./pnuematic hand-held wrapping guns (you

cut, you strip, you pull the trigger ), semi-automatic wrapping

machines and fully automatic numerically-controlled (NC) wrapping

machines.  There is also a wire-wrapping bit made to fit into the

hand-held tools called "Cut Strip and Wrap" which you insert the end

of the wire into and as you make the wrapped connection it cuts the

wire to length, strips off the insulation and makes the wrap. Neat

and quick.  The people we have here who do this for a living like

them a lot.

Advantages:  Quick, relatively inexpensive, easy to repair or make

changes to the boards (unwrap tools are available), well accepted by

industry.

Problems:  Not really suitable for full-scale production because

unless each board is fully automatically wrapped, you have to

consider each board as a new entity; errors may have occurred in the

wrapping of the individual board and finding the errors on one board

won't help you find them on the next - they may not be there or they

may be different.  If a wire goes around a pin along its length

somewhere and is streched too tightly, the corner of the pin may cut

through the insulation, causing a short.  You won't be able to see

it, and even if you do a continuity ckeck of the board, just moving

it around may cause it vanish (temporarily.)  Best advice: use

wrapping vendors of good reputation and be wary of any board that

acts "funny."

Recommendations:  Go for the highest quality everything you can

swing.  The best sockets are made with machined pin/terminals with

spring inserts.  There are a few manufacturers that make boards with

pins permanently swaged into the boards; components plug into one

side of the board, wrapped interconnect is done on the other side.

Real neat.  If you intend to wrap things yourself, you're going to

have to practice.  It looks and sounds easy, but it's not. My hat is

off to the people who can do it 8 hours a day and go home and NOT

beat the kids and kick the dog.

Vendors:      Augat              Boards, sockets, pins.

Mupac              Boards

Garry              Sockets and pins

Slit-n-wrap:

Forget it.  A cheap version of wire-wrap where the extremely thin

insulation is slit by the tool as the wire is (always) hand-wrapped

around the pin.  Unfortunately, the insulation is also slit by every

other blasted thing on the board.  "The hobbyist's wire-wrap."  I

don't know of anyone, hobbyist or not, who has used it more than

once.  Once was more than enough.

Solder Wrap:

More of the same.  Extremely fine wire (32-36 gauge) covered with yet

thinner insulation and a coating of solder.  Wrap the wire around the

pin, cut it off, touch it with a soldering iron, and VIOLA!...a mess.

Basically non-reworkable, produces shorts if you look at it cross-

eyed. Save your money for beer and pizza or something better.

Scotch-somethingoranother:

This travesty was foisted on us by 3M, makers of cellophane tape,

Velcro, and numerous inhalable adhesives.  The sockets were a two-

piece arrangement where you install the body on the top of the board

and a tulip contact on the bottom.  The interconnect was made by

forcing solid insulated wire into the appropriate contacts, just like

telephone patch boards. 2 or 3 insertions was about the limit; after

that the wire just fell out, or worse, became intermittant.  Quick,

easy, fun, and junk.  Requires special boards, special sockets,

special contacts, special wire, and special tools.  Guess who you buy

this stuff from.

Multi-Wire:

A very nice system.  An epoxy-glass board is drilled to accept the

required component leads, plated thru-holes are made, and connections

are made to power and ground planes made of copper (0.001" or 0.002"

thick.)  A layer of adhesive is screened on and very fine wire (36

gauge) is routed to the appropriate holes to form the signal

interconnect.  Routing can be done on either or both sides.  After

the wire is routed, a solder mask is screened on and then the whole

assembly is cured to harden the adhesive and the mask. Now you have

the wiring sandwiched between the board and the solder-mask and held

in place by the adhesive.  Sockets can be used or contacts can be

installed in the holes or the components can be inserted directly

into the holes, and the board can be wave soldered.  Neat and clean.

The board is re-workable and the wiring can be impedance-controlled

for high-speed work. (I know of a 100K ECL system running at 100MHz

that uses Multi-Wire boards.) Twisted-pair conductors can be used.

Problems:  Not cost effective at small or very large volumes.  Enjoys

a middle position.  Probably best at the 500-piece range.  If you're

going to build 100 - 10,000 of something, talk to Multi-Wire.

Vendors:       Multi-Wire (A Division of Kollmorgen Corporation)

Hitachi  (A licensee of Multi-Wire, apparently)

Augat  (they call theirs Unilayer-II)

Printed Circuits:

The old standby.  With good reason.  Nothing can beat the performance

and reliability of printed wiring.  Unfortunately, it is not without

its problems.  Be prepared to pay a few thousand dollars to have

artwork made to have 10 prototypes built, at another few thousand

bucks, to find what is wrong with the artwork so you can start over.

If you have to build more than 1000 of almost anything and you aren't

using PC's (NOT Personal Computers -- Printed Circuits!), you need to

re-evaluate what you're doing.  I have had printed circuits built

that were double-sided, with plated thru-holes, gold-plated edge

connectors, with solder plating, about 6" X 8" that were less than

$20 in small ( q < 10 ) quantities.  If you can create your own

artwork and supply the fabrication shop with all the required photo-

tooling and NC tapes, the cost of a PWB can be disgustingly

reasonable.  In a university environment, a small PC shop could be

quite easily set up and support the needs of the CS, EE, Physics,

Industrial Engineering, and Physical Chemistry departments. Costs

increase with several factors.  If you need more density, you may

need more layers.  More layers calls for more dollars.  If you need

controlled impedance wiring, or fine-line lithography, or heavy power

planes, or strip-line structures, etc. etc., you get to pay more. All

in all, PWBs are the best solution when you can afford them and your

volume requires them.  The more you buy, the more you can afford, of

course.

Stitch-Wiring:

This works almost like wire-wrap, except that the wiring is not

wrapped around a square pin but is spot-welded to the flat end of a

cylindrical pin.  The wiring is run continuously from an inverted

cone-shaped "capillary" tool which allows you to locate the wire with

respect to the top of the pin.  A foot pedal then brings the tool

into contact with the pin and an electric discharge is triggered.

The pressure cuts through the insulation and the discharge welds the

now-exposed conductor to the pin. The tool can then be raised and

moved to the next pin in sequence, or the wire can be severed and a

new "trace" begun.

Advantages:  The method is fast, reliable,  and results in a board

with a very low profile.  The wiring is reworkable, but not very

often on the same pin.  Used a lot by the military.

Problems:  The equipment is pretty scarce, the pins are costly (gold

plating), and each time you have to pull a wire from a pin, you tear

off some of the plating.  This limits the number of times a wire can

be welded to the pin. If you have access to such a machine and the

required sockets and such, it's hard to beat for building one or two

of something.  Otherwise, it's out of reach for us humans.

Genuine Point-to-Point Wiring:

If you have to build one (1, unity, 1 each, only 1) of something,

it's really hard to beat point-to-point.  This is especially true if

it's the type of thing that is relatively simple and the design is

stable.  Put some sockets on a piece of Vector board, get a handful

of parts, some red, blue, green, yellow, black, and white wire (22

gauge, solid), some solder, wire cutters and strippers, band-aids,

can of Pepsi, beer, or spring water, bag of M&Ms, bag of Fritos,

trash can, aspirin, and a soldering iron and go build yourself a

circuit.  Keep a continuity tester around to see how you're doing as

you go along.  If you can get the so-called "Pad-per-hole" type of

board (has a copper pad surrounding every hole on the board, but

isolated from all other pads) you can solder the sockets right into

the board and then wire the leads to each other as required.  It may

not be neat, but it is the ultimate in reworkability and flexibility.

I have seen hand-built audio gear that sold for fabulous prices and

18GHz millimeter wave communications gear built this way. If you're

careful, it can rival PWBs in performance and reliability. Highly

labor-intensive, hardware hackers love it.  Most industry types

wouldn't think of it.  Radio amateurs have been using it for as long

as there have been radio amateurs.

Proto-Boards:

These aren't really a wiring methodology, but they fall into the

realm of hacking and cut-and-try design.  They are plastic blocks

with holes arranged in a 0.100" X 0.100" matrix that you push

component leads down into.  The holes are usually grouped into sets

of 5, so you get 4 common tie-points for every component lead.  You

then wire your circuit elements together with 20 or 22 gauge solid

wire.  You can change anything and everything.  Once you get the

circuit behaving like you want it, you go to one of the above

methods.  Or, if it's REALLY one of a kind, or a one-shot lab

experiment, you tear it down and build something else on the board.

Obviously, it's not a production, or pre-production, or prototype

method; it's sort of pre-prototype.  It's very useful, however, and

makes experimentation quite simple.  Proto-Boards are to hardware as

Forth is to software.  Look for them in hardware hobbyist catalogs.

Basic Recommendations:

For Experimenting:  Proto-Boards, point-to-point.

Building 1: Point-to-point, wire-wrap

Building 2: Point-to-point, wrap-wrap

Building 10: Wire-wrap

Building 100: Wire-wrap (as long as someone else is doing the wrapping),

begin investigating Multi-wire or printed wiring

Building 500: Multi-Wire or PWB

More than 1000: PWBs

General Comments:

Regardless of the method used, the most common error made is lack of

attention to power and ground supplies to the board.  The rules of

thumb are simple: 1) Heavy gauge conductors, as many as practical  2)

Plenty of decoupling capacitance to keep the power supply lines quiet

3) Low inductance power supply leads 4) Well distributed power and

ground wiring.

Make a wiring list and spend some time doing the physical design of

your circuit.  I know this sounds trivial but it's easy to turn the

whole project into so much spaghetti without a little planning.  It's

also easy to turn the board into a near work of art with planning.

Use up a few sheets of a quadrille pad, and do it right.

Failure to observe any or all of these rules will only cause grief.

I saw a memory board where the write enable pulse vanished too soon

to ever write data into the memories due to ignoring 2 & 3.  On the

other hand, I've built wire-wrapped memory boards (BIG memory boards

- 22" X 16") that accessed in less than 35 nsecs and the pulses were

as clean as mountain water.  But I followed the rules.

Some Addresses:

Multiwire Division

Kollmorgen Corporation

3901 East La Palma Ave.

Anaheim CA 92807

714 632-7770

Hitachi Chemical Company America Ltd.

1333 Lawrence Expressway  Ste. 265

Santa Clara  CA  95051

408 244-2570

Augat Interconnection Systems Group

40 Perry Avenue

P.O. Box 1037

Attleboro  MA  02703

617 222-2202

OK Machine and Tool Corporation

3455 Conner St.

Bronx  NY 10475

212  994-6600

Vector Electronic Co. Inc.

12460 Gladstone Ave.

Sylmar  CA  91342-0336

818 365-9661

3M/Electronics Products Division

225-1 3M Center

N Saint Paul  MN  55144

612 733-7408

Books:  Bell Labs once published a 3-Volume set called "The Physical

Design of Electronic Systems".  It's great.  (I don't know anyone who

can understand it, but it must be great.  It's from Bell Labs.)  It's

also not the book for stuff like this.

Most of this stuff is half magic and half instinct.  I would

recommend trying to get subscriptions to trade journals.  Some of the

reasonable ones are "Electronic Packaging and Production", "Circuits

Manufacturing", "Insulation Circuits", "Connection Technology",

"Electri-Onics", "Micro-electronic Manufacturing and Testing", and

"Hybrid Circuit Technolgy".

There *is* a book called "Microelectronic Interconnection Methods" or

something like that around.  I think it was written for your basic

hobbyist ($9.95 paperback with slick front cover art having nothing

much to do with microelectronics or interconnections...)  I'll try to

track it down and determine real title and source.  That's the only

potentially reasonable book I can think of.  Unfortunately, not a lot

gets written concerning this topic.  Not very glamorous, I suspect.

More money to be made writing "Yet Another, Still Easier, Faster

Better, Effective, Upwardly-Mobile Guide to Using Lotus 1-2-3 (tm)

for The Management-Type with The Intellect of Your Average Crowbar."

Sigh.

'brd

--

Larry J. Huntley         Burroughs -(B)- Corporation

Advanced Systems Group   MS-703

10850 Via Frontera   San Diego, CA  92128

(619)  485-4544