Date: Thursday, 16 May 1985  07:29-MDT

From: Jeff Skot

Re:   Z800 preliminary specs

There is much talk about the Z800.  I thought that I'd inject some

facts straight from Zilog to substanciate the excitement.

The following is taken from the

"Z800 MPU Family Preliminary Product Specification" September 1983

00-2259-01

from	Zilog, Inc

1315 Dell Ave

Campbell, California 95008

(408) 370-8000

TWX 910-338-7621

FEATURES:

. enhanced Z80 instruction set that maintains object-code compatability

with Z80 microprocessor

. on-chip paged Memory Management Unit (MMU)

. large memory address space: 512K and 16M byte versions

. On-chip, high speed local or cache memory

. high performance 16 bit Z-BUS interface or 8-bit Z80-compatible bus interface

. four on-chip 16 bit counter/timers

. four on-chip DMA channels

. On-chip full duplex UART

. 10-25 MHz CPU processor clock

chip #	# pins	data path	address space	on-chip peripherals

Z8108	40	8 bits		512 K		no

Z8116	40	16 bits		512 K		no

Z8208	64	8 bits		16 M		yes (there are enough pins

Z8216	64	1 bits		16 M		yes  left on the package)

Central to the Z800 microprocessor is an enhanced version of the Z80 CPU.

To assure system integrity, the Z800 microprocessor can operate in either user

or system mode, allowing protection of system resources from user tasks

and programs.  System mode operation is supported by the addition of the

system Stack Pointer to the working register set.

I/O address space has been expanded by the addition of an I/O page register

used to select pages of I/O addresses.  The 8 bit I/O Page register can

select one of 256 possible pages of I/O addresses to be active at one

time, allowing a total of 64K I/O addresses to be accessed.

[ The I/O instructions are still using 8 bit addresses so this

register is appended to form a 16 bit I/O address ]

There are 256 bytes of on-chip memory present on all members of the Z800

family.  This memory can be configured as a high speed cache or as a

fixed address local memory.  When configured as cache, the memory can be

programmed to be instruction only, data only, or both data and instructions.

The cache memory allows programs to run significantly faster by reducing

the number of external bus accesses.  Operation and update of the cache

is performed automatically and is completely transparent to the user.

When used as a local memory, the addresses are programmable, allowing

"RAMless" systems to be used.

All members of the Z800 family contain an on-chip clock oscillator.  Also

present is a refresh controller that provides 10 bit refresh addresses for

dynamic memories.

The 64 pin versions of the Z800 MPU contain additional on-chip peripherals

to provide system design flexibility.  To support high-bandwidth data

transmission, four DMA channels are incorporated on-chip.  Each DMA

channel operates using fill 24 bit source and destination addresses with

a 16 bit count.  The channels can be programmed to operate in single

transaction, burst, or continuous mode.  System event counting and timing

requirements are met with the help of the four 16 bit counter/timers.

The countdown/timer functions can be externally controlled with gate and

trigger inputs, and can be programmed as retriggerable or nonretriggerable.

Also, a full duplex UART, capable of handling a variety of data and

character formats, is present to facilitate asynchronous serial communication.

An additional feature of the 16 bit bus interface is the ability to support

"nibble-mode" dynamic RAMs.  Using this feature (known as burst-mode),

the bus bandwidth of memory read transactions is essentially doubled.

Burst mode transactions have the further benefit of allowing the cache

to operate more efficiently by guaranteeing a high probability that the

contents of the associated memory will be present in the cache.

The Z800 CPU architecture supports four distinct address spaces ...

. CPU register space

. CPU control and status register space

. Memory address space

. I/O address space

Status and control registers:

There are ten status and control registers available to the programmer

in the Z800 MPU.  Table 1 shows the addresses occupied by the

registers in the status and control register addressing space:

control register name		address (hex)

====================		===========

Bus timing and Control		control 02

Bus timing and initialization	control FF

Cache control			control 12

Interrupt Status		control 16

Interrupt/Trap Vector Table	control 06

I/O Page Register		control 08

Local Address Register		control 14

Master Status (MSR)		control 00

Stack Limit			control 04

Trap Control			control 10

Interrupt/Trap Vector Table:

The format of the Interrupt/Trap Vector Table consists of pairs of

Master Status register and Program Counter words, one pair for each seperate

on-chip interrupt or trap source.  For each external interrupt, there is a

separate Master Status register word and Program Counter word (for use if

the input is not vectored).  If the external interrupt is vectored, a vector

table consisting of one Program Counter word for each of the 128 possible

vectors that can be returned for each input line is used instead of the

dedicated Program Counter word; thus for vectored interrupts, there is only

one Master Status register for each interrupt type.

INTERRUPT/TRAP VECTOR TABLE

address		contents

=======		========

00	unused

04	NMI Vector

08	Interrupt Line A Vector (End of Process)

0C	Interrupt Line B Vector

10	Interrupt Line C Vector

14	C-T0

18	C-T1

1C	C-T2

20	C-T3

24	DMA0 Vector

28	DMA1 Vector

2C	DMA2 Vector

30	DMA3 Vector

34	UART Receiver Vector

38	UART Transmitter Vector

3C	Single-Step Trap Vector

40	Breakpoint-on-Halt Trap Vector

44	Division Exception Trap Vector

48	Stack Overflow Warning Trap Vector

4C	Page Fault Trap Vector

50	System Call Trap Vector

54	Priviledged Instruction Trap Vector

58	EPU <- Memory Trap Vector

5C	Memory <- EPU Trap Vector

60	A <- EPU Trap Vector

64	EPU Internal Operation Trap Vector

68 - 6C	reserved

70-16E	128 Program Counters for MNI and Interrupt line A Vectors

(MSR from 04 and 08, respectively)

170-26E	128 Program Counters for Interrupt Line B Vectors

(MSR from 0C)

270-36E	128 Program Counters for Interrupt Line C Vectors

(MSR from 10)

***** my own observations *****

Despite the system/user modes, they still have 'primary' and 'auxillary'

sets of registers.  The only register duplicated for system/user

is the SP (Stack Pointer).  A whole other set of registers for system

would've been nice.  The only way to switch register banks is with

EX (exchange Accumulator/flag with alternate bank) and

EXX (exchange byte/word registers with alternate bank).  I think

this limits the use of the registers since it is easy to forget which

is in use (there is no way to see which is in use or say 'use primary',

only 'use other').  EX and EXX are NOT priviledged instructions so you

cannot reserve their use for the system mode.

Priviledged instructions consist of the CPU controls, I/O and memory

control.

It would be nice if this ever came to pass, but Zilog is busy with

their Z80,000 32 bit processor so this takes a back seat.  Sigh.

Jeff 'armed to the teeth in knowledge' Skot

at midtown Somerset, New Jersey

{ ihnp4 | mcnc | cbosgb} abnji ! jeff

#############################################################################################################