Date: 4 Jul 88 13:16:38 GMT

From: pnet01!pro-sol!mdavis@nosc.mil (Morgan Davis)

Subject: 4K XMODEM EXTENSION REFERENCE

THE 4K XMODEM EXTENSION REFERENCE

=================================

A significant but simple upgrade for 1K XMODEM

Developed by

Morgan Davis

Document Revision 1.00

July 4, 1988

Distribute as widely as possible.

CONTENTS

========

I. INTRODUCTION

General Description and Background........ 1.1

Features.................................. 1.2

Why 4K?................................... 1.3

II. PROTOCOL SPECIFICATIONS

Symbol Definitions........................ 2.1

Initial Handshaking -- Receiver........... 2.2

Initial Handshaking -- Sender............. 2.3

The Transfer.............................. 2.4

Handling Errors........................... 2.5

Buffer Considerations..................... 2.6

Longer Timeout Option..................... 2.7

III. APPLE-SPECIFICS

ASCII Express "Pro" Extension............. 3.1

AE Pro Mode Compatibility................. 3.2

ACKnowledgments

Feedback and Line Noise

PART ONE: INTRODUCTION

======================

1.1:  General Description and Background

This document describes the 4K (kilobyte) extension to 1K XMODEM protocol.

1K XMODEM is a widely used and accepted standard in transferring information

between terminals and communications equipment.  This document does not,

however, delve into the details of XMODEM protocol.  Plenty of documentation

has been written about XMODEM and its number of head-spinning variations.

Perhaps the most complete, yet concise reference describing this protocol is

by Chuck Forsberg who improved upon XMODEM by developing YMODEM, a 1K

variation of the standard 128-byte packet used in XMODEM.  Forsberg's

_XMODEM/YMODEM_PROTOCOL_REFERENCE_ is widely distributed and available on

most information services and local bulletin board systems.  If you can't

locate a copy for yourself, write to:

Omen Technology, Inc.

17505-V Sauvie Island Road

Portland, Oregon  97231

503/621-3406

or contact Chuck Forsberg at one of these addresses:

Compuserve: 70715,131

UUCP: tektronix!reed!omen!caf

Telegodzilla BBS: 503/621-3746

1.2:  Features

Transferring a file with large data packets means it takes less time to

send than if small packets were used.  This is because after each packet

is sent, the sender must wait for an acknowledgement from the receiver.  This

acknowledgement tells the sender "everything is fine" and to continue sending

the next packet.

An ideal delay between packets (with no disk-access) is usually less than 200

milliseconds, about 1/5th second.  But the delay can vary depending on

environmental factors.  Most significant is the type of connection between

the sender and receiver.  If a packet-switched network is utilized, such as

Telenet or Tymnet, this delay can be quite lengthy -- from a half second on

up to five seconds, perhaps longer.  Even if two modems are connected

directly through the phone system delays can also occur, especially with

error-correcting modems like the USRobotics Courier HST 9600.  Buffered

multi-user systems are also a source of unwanted delays.  You can't

arbitrarily speed up your modem to get around this problem, but you can make

use of a larger data block size through software.

For example, if a 32K file is being sent using 4K data blocks, only eight of

these inter-packet delays takes place during the course of the transfer.  But

if a smaller packet is used, say the standard 128-byte size employed in

regular XMODEM, 256 acknowledgements would transpire.  If it takes one second

between each packet for this brief chat, you save about 4.25 minutes using a

4K data block as opposed to a 128-byte block, at the SAME baud rate.

As the size of the file increases, so does the improvement factor when larger

data blocks are used.  In addition, the faster the transfer rate (2400 bps

and higher), the better the performance with a larger block.

1.3:  Why 4K?

Using larger packets in XMODEM is nothing new.  This was realized as soon as

1200 bps modems became popular.  The step up from 128-byte packets to 1K

packets became even more significant as 2400 bps modems became affordable.

But now, with error-correcting 9600 bps modems and a wider use of

packet-switched networks like Telenet's PC Pursuit, the 1K data block is no

longer an optimal method.

If a larger data block is better, why not use

something really big, like a 16K packet?

Three reasons:

One.  To be widely accepted and compatible with most microcomputers, a

dedicated 16K packet buffer would bestow a major burden on most terminal

programs, especially on 64K-RAM machines.  The 4K packet proves to be a

significant improvement over smaller sizes while being reasonable in its

demands on most machines.

Two.  Having to resend just one 16K packet due to an error at 2400 bps or

lower (very popular speeds at the time of this writing) would nullify any

improvement in using a large packet.  Even 8K would be unreasonable.

Three (and this is a technical problem).  Many computers just aren't fast

enough to handle a constant 16K stream of data at speeds faster than 2400

bps.  As each character is received, the computer might have to service more

than one interrupt, manage buffering, handle events from the user, as well as

perform CRCs and other calculations.  A lot of computers just couldn't keep

up since there is no software flow control during an 8-bit, asynchronous

transfer.

Data communications involves transactions between many different types of

computers.  So when 19,200 bps speeds and one megabyte RAM machines become

standard issue, we will have definitely outgrown the 4K packet size and can

consider something more robust.  But until that time . . .

PART TWO: PROTOCOL SPECIFICATION

================================

2.1:  Symbol Definitions

<SOH>  = 01h = Start of header (128-byte block)

<STX>  = 02h = Start of header (1K block)

<EOT>  = 04h = End of transfer

<ACK>  = 06h = Acknowledge (good block)

<NAK>  = 15h = Negative ACK (bad block)

<CAN>  = 18h = Cancel transfer

<C>   = 43h = CRC request character

<K>   = 4Bh = 1K data block request character

<L>   = 4Ch = 4K data block request character ("L" for "Large").

<SSTX> = 82h = Special Start of header (4K block)

2.2:  Initial Handshaking -- Receiver

To transfer a file with 1K or 4K data blocks, the receiver must request the

CRC-16 option, as a cyclic redundancy check is required to use 1K and 4K

packets.  This is done by sending <C> every three seconds instead of <NAK>.

The sender will look for a <NAK> or <C> from the receiver as a cue to begin

the transfer.

Though not required but as a consideration to the sender, the receiver

should send <C><K><L> rather than just <C>.  This tells a 4K-compatible

sender that 1K and 4K blocks can be handled by the receiver.  (The sending of

<C><K> is an accepted standard for requesting 1K blocks).

The order of the letters <K> and <L> shouldn't be significant to the sender,

but they should be sent as <K><L> as a matter of convention.  This will avoid

any problems if 4K blocks are requested from a sender that only understands

128-byte and 1K data blocks, as it will see <C><K> and ignore the following

<L>.

The receiver should send these request characters every three seconds and

wait for either <SOH>, <STX>, or <SSTX> to come in from the sender.  These

characters denote the start of a 128-byte, 1K, or 4K packet being delivered,

and CRC-16 is assumed.

If the receiver has sent the request characters three or four times and

nothing has come back from the sender, it reverts to standard XMODEM and

begins sending <NAK> once every ten seconds for at least a minute.  At this

point, the receiver will only recognize 128-byte, checksum packets.  It

ignores any start of header character other than <SOH>, as anything else

could be garbage from line noise.

Should the receiver get two <CAN>s in a row while waiting for the start of a

packet, it can abort the receive attempt.

2.3:  Initial Handshaking -- Sender

If the sender gets a <NAK> while waiting to begin the transfer, it knows that

a standard, 128-byte, checksum XMODEM is requested and can begin sending

right away.

If the sender gets a <C>, it prepares to use the CRC-16 option.  At this

point, it should set up a one-second timeout and wait for either a <K> or

<L>.  If a <K> is detected, it should wait one more second for the

possibility of an <L> to come in.  To effect this in an elegant manner, a

loop can be created which simply waits one second per character until a

timeout occurs.  When a character comes in it is examined to see if it is a

<K> or <L> (the other shouldn't matter) and, if so, the appropriate flags are

set.  It then loops back for more characters.  When a timeout occurs, the

loop terminates and the transfer begins immediately.

With some implementations, the receiver will only send a <C> and the block

size is already assumed to be 1K or 4K.  Therefore, the sender should begin

sending blocks using a size specified by the operator of the sending program.

Should the sender detect two <CAN>s in a row, it can elect to abort the

transfer.

2.3:  The Transfer

After each side has introduced itself, the transfer begins.  At this point,

all the rules that apply to standard 128-byte and 1K XMODEM are applicable to

4K mode as well.  The only difference is that a 4K packet begins with <SSTX>,

which simply is <STX> with bit 7 set to 1.  This alerts the receiver that

4096 bytes of data are contained in the data block of the packet being sent.

The transfer continues as you would expect.

When the sender nears the end of the file, it ought to begin tapering down

the data block sizes using the "best fit" logic.  For example, if there is

less than 4K left to send, it should fall back to 1K or 128-byte blocks,

whichever is most efficient.  This will insure that the resulting file will

not contain any more than 127 padding characters (usually nulls, 00h).

Imagine the great injustice to the receiver if nearly 4K of unnecessary

padding was delivered.

2.4:  Error Handling

Bad block retries due to errors can really degrade performance, especially

with 1K and 4K blocks.  So it is recommended that the implementation of the

sender include "fall-back" and "step-up" block size logic.

A fall-back occurs after too many consecutive errors.  This causes subsequent

data blocks to be sent out using the next smallest size block (i.e. from 4K

down to 1K, and from 1K down to 128-bytes).  If the current block size is 4K,

only two consecutive errors should be allowed before the sender falls back to

1K.  At 1K, three consecutive retries are allowed, and then 128-byte blocks

are used.

A step-up occurs after a run of successful packets are sent, usually eight or

more.  In a software implementation, this could be known by incrementing a

counter each time a packet is successfully sent.  Should the receiver ever

NAK a packet, the sender must reset this counter.

The sender must also maintain a variable that defines the largest block size

allowed so that it cannot step-up to a size that the receiver is not prepared

to handle.  In addition, consideration must be given to the exact moment when

stepping up is done.  (This is described in the next section).

If a transfer is cancelled by too many consecutive errors, or by operator

intervention, several <CAN> characters, five are adequate, should be sent to

the other side, followed by an equal number of backspaces (08h) to clean up

the display.

2.5:  Buffer Considerations

To handle a 4K packet, a receiver must allocate at least 4K of contiguous

memory in which to store the data portion of the largest sized packet.

However, since a mixture of 128-byte, 1K and 4K packets can be sent during

the transfer, it is possible to overrun the end of the buffer.  Imagine that

a 4K buffer is already 75% full and a 4K block is sent.  Since there is no

way to know how large the next block will be until it starts coming in, it

could easily wipe out information beyond the end of the buffer.

This situation can occur when a sender employs block size "step-up" logic.

As a consideration to the receiver, the sender should never transmit a 4K

block unless the amount sent so far is a multiple of 4096 (1000h), or zero.

A simple way for the sender to handle this is by performing the step-up

checking during its own 4K file buffer refresh sequence.  This will ensure

that the receiver will be able to handle a step up to 4K without overrunning

its packet buffer.

If the receiver detects a 4K packet coming in at an inconvenient time, the

best way to handle it is to write the previously buffered blocks to disk, NAK

the sender's 4K packet and prepare to receive it again into the empty buffer.

This can severely degrade the performance of the transfer, but it should

never occur if the sender's implementation is done properly.

2.6:  Longer Timeout Option

Timeout errors can occur frequently when transferring files with a

packet-switched network.  A timeout happens when a certain amount of time

passes and no activity has taken place.

Since standard XMODEM implements a one-second timeout during the reception of

characters in the data block portion of a packet, the larger data block size

could cause some delays with hosts that buffer outgoing data.  A host's

buffering-related delay could cause the receiver to time out in mid-packet.

The programmer should allow the user to specify increased timeout durations.

This should be employed during both sending and receiving so that

transferring files over a network, such as Telenet or Tymnet, will result in

fewer timeout errors.

PART THREE: APPLE-SPECIFICS

===========================

3.1:  ASCII Express "Pro" Extension

An adopted standard for transmitting files in the Apple world involves the

exchange of a special packet of file information.  This resource information

is external to the data included in the file and is stored in the file's

directory entry.  Since standard XMODEM makes no provision for sending this

extra but important information, the ASCII Express "Pro" (AE Pro) technique

was developed by United Software Industries, Inc.  Documentation for this

special extension to XMODEM is available from most information services or

bulletin boards.

3.2  AE Pro Mode Compatibility

To maintain AE Pro compatibility with the extended CRC, 1K and 4K modes, the

following guidelines apply:

o   The initial AE Pro exchange is not made until the normal

sender/receiver handshaking is done to determine CRC or checksum,

and requested block sizes.

o   The initial AE Pro packet is sent using these three bytes:

<SOH>+80h (81h), the Apple DOS 3.3 filetype byte (or 00h if

using ProDOS), and a complement byte of the filetype value

when exclusive-OR'd with $FF.

The rest of the transfer ensues as described in the AE Pro Extension

reference.  When the AE Pro file information packet is sent at the end of a

ProDOS-based transfer, it uses a 128-byte data block, and CRC-16 or checksum

depending on how the transfer had been established.

ACKnowledgments

===============

Special thanks to Don Elton for providing the proving grounds with his own

terminal program for this new specification as it was being conceptualized.

Thanks also to United Software Industries, Inc., for letting me develop this

on their time (while getting paid, no less).

Feedback and Line Noise

=======================

Questions and comments about the 4K extension can be sent to:

BIX:       mdavis

MCI Mail:  mdavis

AppleLink: mdavis

ProLine:   mdavis@pro-sol

UUCP:      crash!pnet01!pro-sol!mdavis

ARPANet:   crash!pnet01!mdavis@pro-sol.nosc.mil

InterNet:  mdavis@pro-sol.cts.com

US Mail:   Morgan Davis, PO Box 4313, La Mesa, California 92044

Modem BBS: pro-sol: 619/281-7222; Speeds: 9600, 2400, 1200, 300

Morgan developed the ModemWorks telecommunications language software and has

used it to create ProLine, a world-wide, networked message and conferencing

system.  He has also written several books on how to program Apple computers,

his most recent being "Mastering the Apple IIGS Toolbox", and "Advanced

Programming Techniques for the Apple IIGS Toolbox".  Currently, he is manager

of product development at United Software Industries, an author of the

MouseTalk terminal program, and a founder of Living Legends Software.  Morgan

is also moderator of the Apple conference on the BYTE Information Exchange.

##################