Interference Problems

So you got that new modem and are now hooked up to the BBS. What

is	all this garbage being printed on the screen??? You say I've hooked

up the modem correctly and have the terminal program set for the modem.

NOW WHAT'S WRONG???? Well friend you may have just been bitten by the

phone line or computer noise bug. When it bites it does a job on you

(or should I say your computer). Here are a few hints that may ease the

bite. Be aware that it is doubtful that you can cure all of it. Ma Bell

has tried to for years!

Let's take a look at what line or computer noise is and then maybe

we can knock it down to a bearable level.

Computer noise:

Computer noise comes from a great many places inside the computer

itself. The chips themselves make a certain amount of it. The crystal

which gives us the timing for the clock, the disk drive and causes all

the instructions you or the program give it to move through the

computer correctly generates a great deal of noise. The FCC (Federal

Communications Commission) sets a limit on how much noise

(interference) is allowable. It is a pretty high allowable limit for

home use computers and lower for commercial or heavy use computers.

Here is something to try. Turn on your computer and all other

hooked up equipment. Then bring in a TV or an AM/FM radio. Turn on the

radio or TV and tune through the channels or band. You will soon hear

on certain channels or frequencies a lot of strange sounding noise. If

you begin to turn off your computer equipment the noise will change or

go away. This can and many times is part of the problem. This should be

cleaned up as much as possible before doing anything else. Be aware

that you probably won't get rid of all of it, but you should be able to

knock it down. Let's see what can be done without getting into the

computer.

First be sure you are putting your AC plug into a 3 wire

receptacle that has all three wires hooked up!!! You think this guy is

crazy right?? Wrong!! Many times it looks as if you have three wires

but---get an AC line checker and make sure. Then get a line protector

which protects against spikes and noise. Radio Shack sells them and so

do many other places. ACE Hardware in Altus is one. You can buy them

from advertisers in many of the magazines. By the way, everytime you

change something check the radio or TV to see if the noise you heard

has changed. Don't get discouraged. Sometimes you will have to do

several things to see a difference.

Next get a length of wire and ground ALL of your equipment

together. That means to hook the wire to your compute where there is

metal showing, then to the modem, the printer, vidio terminal, etc.

Make sure you have metal to metal contact. Again check the radio or TV

to see if the noise has dropped. You can also check to see if the BBS

looks better on your terminal.

Now let's try to erase some of noise that can be coming from your

AC power cords. Radio Shack has toroid choke cores (part # 273-104) two

to a package for about $7.00. No I don't work for them. They are

available at the store here, thank goodness. Follow the instructions on

the package and put them on each AC power cord.

When you have done all this you have done about all you can do

without getting into the computer or printer. I hope this will help

solve some of the problem.

Telephone Line Problems

Well you tried all that stuff about the AC line and you still have

problems!! That Blankety Blank guy still hasn't cured MY problem!! I

never said it would be easy and I never said ALL of it could be cured.

Here are some tips on the phone line itself that could cure or at least

knock down the problem.

Do you have a cordless telephone?? If you do is it turned on

and/or hooked up? If it is turn it off and unhook it (YES BOTH) and try

the BBS again. The GE cordless with PLL that was sold at Altus AFB WILL

cause problems. I'll sell one cheap!!

Do you have only one phone number in your house? I don't care how

many phones, just the numbers. Be aware that the phone company brings

in 4 wires, but only two plus a ground  are used for a single phone

number. AHA-trouble right? Right! The unused line can act as an antenna

and pick up noise or RFI (radio frequency interference). Can this be

cured? Yes to a certain degree. How? Well here we go----.

Telephone wire can break down from age, temperature extreems (we

sure have that here) and from movement (twisting, being hit, going

through walls, +++). The best thing to do is to instal new wire to

where you will use the modem and NOT have any phone on that line. You

can buy installation kits almost anywhere and it is very easy to do it

yourself. One thing, never use staples to mount the wire to the wall as

you run it to the modem. They can break the wire. There is a plastic

clip (KMart and WalMart sells it) with a nail at one end. This is a

much better way and will cause less problems. When you hook up the wire

to the phone block outside the house be sure you remember which wire is

ground. It's marked. You may want to use it later.

If you do the above or decide your wire is ok then you may want to

try this. Turn on your computer and modem plus all other items you want

on. Then while in terminal mode type AT M2 L . M2 should keep your

speaker on all the time and L should set the volume to low so you can

stand the noise. Now access the BBS and watch the terminal and listen

for popping noise. If you hear noise (besides the normal sound the

modem makes) you should see garbage printed out at the same time. If

you see the garbage or hear noise then shut down, use AT M1 to reset

the speaker to default. Then go outside to the phone box. Check what

wires are being used. In my case the red and green were, with yellow as

ground. The black wire coming into the house was not even connected.

Strip a little of the plastic from the wire that is not being used and

connect it to ground. Then check the modem as before. Less popping and

less garbage?? I hope so. If not try this as just about the last

resort. Go to Radio Shack and get two small ceramic capacitors. The

value should be .001 (272-126) to .0047 (272-130). These should work

with no problem. Now remove the cover on the connector block where your

modem is connected to the phone lines. Connect one wire on each

capacitor to each of the wires that are in use (the ones you checked on

the outside box). Connect the other wire on the capacitor to  the

ground wire connection. In my case I connected a capacitor between red

and yellow, then another between green and yellow. Replace the cover

and try the BBS again. Gets tiring doesn't it?? The only other thing

you can try is to use a toroid choke core on the phone line going to

the modem. Do it the same way as you did the AC cords.

This is about all you can do to your equipment and SHOULD cure

most of the problem. I sure hope it does!!!

There are two other items I use on the phone line, a ringer

silencer (RS #43-127) $7.95 and a spike protector (RS #43-102) $12.95.

These are not really needed but they are sure nice to have.

If you have a question on interference I'll try to answer it for

you. Please understand that I am not an expert. I'm just a ham radio

operator who like computers and has had problems with interference.

GOOD LUCK

Robert L. Willsey

Box 10

Martha, OK 73556

OR

BobWill (#27)

Majik Shoppee

405-482-2536

######################