FLOPCOPY.ASM     FLOPPY DISK COPY PROGRAM USING 1 DRIVE AND A HARD DISK

WRITTEN BY G. YOUNG, PO BOX 3218, NORTH HOLLYWOOD, CA 91609

FLOPCOPY IS A PROGRAM USED TO COPY ONE FLOPPY DISKETTE TO ANOTHER FLOPPY

IN SYSTEMS THAT HAVE ONLY ONE FLOPPY DRIVE BUT HAVE A HARD DISK.  IT WORKS

AS FOLLOWS: READ THE DIRECTORY OF THE FLOPPY DISK USING THE MASK IN THE COMMAND

TO CREATE A LIST OF FILES TO BE COPIED (EXAMPLE: FLOPCOPY *.ASC TO COPY ALL THE

ASC FILES FROM ONE FLOPPY TO ANOTHER).  THEN, USING THE LIST IN MEMORY, COPY

THE FILES FROM THE FLOPPY TO THE HARD DISK.  IF THE FILE ALREADY EXISTS ON THE

HARD DISK, ASK IF YOU WANT THE FILE COPIED OVER WITH THE FLOPPY VERSION AND

SET AN INDICATOR IN THE FILE LIST THAT THIS FILE WAS ALREADY THERE.  THEN THE

PROGRAM WILL REQUEST THE BLANK DISKETTE THAT WILL BE THE DESTINATION DISK TO BE

MOUNTED.  USING THE FILE LIST NOW IN MEMORY, THE FILES ARE THEN COPIED FROM THE

HARD DISK TO THE BLANK FLOPPY.  WHEN ALL FILES HAVE BEEN COPIED, THE PROGRAM

WILL ASK IF ADDITIONAL COPIES ARE TO BE MADE.  IF NOT, THE FILES ARE DELETED

FROM THE HARD DISK.  IF THE FILE WAS ALREADY ON THE HARD DISK AS DETERMINED

BY INDICATOR SET PREVIOUSLY, THE FILE WILL NOT BE DELETED.  THE PROGRAM IS

SMART ENOUGH TO DETECT A FULL DISKETTE CONDITION AND A COMMAND WITHOUT

A MASK.