FROM: CAMERON W. COTRILL

TO: ALL ZSYSTEM SOFTWARE DEVELOPERS

Revised 01/09/88  Cameron W. Cotrill

BANKED CP/M 2.2 COMPATIBLE OS

With the advent of the 64180 and Z280, having more than 64k

of memory has become easy.  Prior to this, there have been ways

of bank switching a Z80 system, and most use similar methods -

low memory is swapped while high memory is retained.  Despite the

fact that banking methods used are similar, nobody has proposed

any details of extending the CP/M 2.2 and ZCPR3 environment to

support banked memory.  This document is a proposal to set forth

the hardware and software interfaces to support such a system in

a manner that is consistent with the hardware independence of

ZCPR3 and CP/M 2.2.

The starting point for this definition is a Z80 system using

ZCPR3 and ZSDOS, and supporting at least 2 banks of memory.  In

requiring the ZCPR3 environment specification, more information

on the system is available and the OS can take advantage of this.

One important application of the environment (and required TCAP

definition) is enhanced line editing is possible using BDOS

function 10 (A superset of CP/M+).  The address that banking

takes place is not important, but must be a page boundary between

8000h and 0F000h.  Most banked Z80 systems fit this

specification.

In order to support banking, the entire operating system must

be altered.  Naturally, backward compatibility with existing

programs and utilities must be maintained to the maximum degree

possible.  This means that CCP, BDOS, and BIOS entry points must

be maintained as in CP/M 2.2, and all functions supported by CP/M

BDOS and BIOS must also be maintained.

I. MEMORY MANAGEMENT

Currently, extended ram (above 64k) is used for RAM disk on

most systems that have it.  A few bank part of the BIOS to allow

for larger TPA's, but no uniform specification to use this memory

is yet available to applications programs.

In order to allow applications programs to use banked memory,

system independent memory management routines must be added to

BDOS.  Naturally, the hardware dependent parts of bank switching

must be handled by new BIOS entry points.  The necessary added

routines would spill BDOS over the 3.5k space it has allotted to

it.  However, since we are banking the system, the overflow can

be stuck in a reserved bank and swapped in and out as needed.

The same follows for BIOS.

To facilitate this, we will reserve bank 0 as the system

bank. The normal TPA will be located in bank 1, with banks 2-nnnn

available as extended memory for the application or the system.

Starting at address 0 in the system bank is the Banked BIOS area.

This area is 8k long and is reserved for BIOS (and boot ROM if

desired).  Above this, at address 2000h, is the banked BDOS area.

This reserved area is also 8k.  At 4000h is the 2k CCP buffer.

The BIOS should load CCP from this buffer instead of from disk.

BDOS is never re-loaded.  The memory above CCP BUFFER (starting

at 4800h) and extending up to the bank boundary is used for

system extensions.

System Bank

-----------

Z3ENV

Z3 BUF                        Must not be banked

RES BIOS

RES BDOS (3.5k)

CCP (2k)

-----------

TOP TPA                       May be banked

-----------

SYS EXTEND

CCP BUFFER (2k)               Must be banked

BANKED BDOS (8k)

BANKED BIOS (8k)

It is important to note which addresses are fixed in the

system and which addresses are not.  The addresses of Banked

BIOS, Banked BDOS, CCP Buffer, and the starting address of SYS

Extend are fixed.  All other addresses in the system may be

adjusted as required by the system implementor, so long as

modules with specified sizes (CCP, BDOS) are kept to the proper

size and in order.

In order to provide maximum utility for programs using

banking, the following types of services must be provided by the

OS -

1. Allocate bank to application

2. Free bank

3. Call routine in bank nnnn

4. Jump to routine in bank nnnn

5. Get data from bank nnnn

6. Store data to bank nnnn

7. Get bank size

8. Get/Set DMA bank (for disk ops)

This set of functions will allow a program to use banked memory

for either program memory, data memory, or both.  No restrictions

are placed upon how banks of memory may be requested by the

application.

For the purposes of this document, the term application

shall be defined as a transient program, RSX, or Device Driver.

In order to prevent undue restrictions on memory size, the

bank number will be a 16 bit unsigned number.  This will allow up

to 65536 banks of memory (about 3.14 gigabytes if 48k banks are

used).  This should be enough memory capacity for a while.

Physical memory need not be this big, nor does BIOS need to

allocate the banks in sequential order.  BDOS will call the BIOS

alloc entry point to get a bank of memory. In order for BIOS to

keep track of what application owns what memory, a task number

will be passed to BIOS in BC.  If a bank is available, BIOS will

return the bank number in HL.  If no bank is available, 0 is

returned in HL.  Note that 0 is a valid bank but is never used by

an application as it is reserved for the system.  It is proposed

that task numbers 0-3f be reserved for BIOS.  All other task

numbers will be assigned by BDOS.

Since BIOS handles the details of memory allocation, this

means BIOS may allocate memory to itself.  Note that since BIOS

takes care of the physical details of banking, it is possible to

support virtual memory for banking.  This is probably best used

with the Z280 which has direct MMU support for this, but could be

used with any system.

Note that the 2k used by CCP must not be banked.  The primary

reason for this is to allow banked programs a guaranteed 2k of

global memory that can be used for data and program common to all

banks.

Upon termination of an application, all banks used by the

application will be freed by OS.

Note that what is required for BDOS/BIOS compatibility is an

offset of 0DFAH from BDOS entry point to BIOS Warm Boot.  If the

resident portion of BDOS is under 0E00H in length, the remaining

space may be used for system segments or BIOS.

II. SYSTEM EXTENSIONS

System extensions are additions to either BDOS or BIOS that

can define new functions or alter existing functions.  A Device

Driver (DD) alters BIOS functions and a Resident System Extension

(RSX) alters BDOS functions.  As such, the DD is hardware

dependent and the RSX is hardware independent.  Both are

relocatable modules that are loaded into the System Extension

Area on the system page or into an alternate bank if the system

area is full.

Several new BDOS calls need to be provided to manipulate

these modules.  The modules themselves have a similar structure.

Pertinent information about the module is kept in the module

header.  Details of the added BDOS calls and the module header

structure need to be defined.

III. ENHANCED BDOS FUNCTIONS

Several enhancements to the internal operation of BDOS are

proposed:

1. Hashed directory buffer to speed unambig file directory

searches.

2. Multisector I/O support.

3. Enhanced Function 10 including last line recall and

insert/overwrite editing.

4. I/O redirection.  The following capabilities are

suggested: Unix-like STDIN, STDOUT, STDERR logical device

assignment, re-assign RDR: and PUN: as single AUX: device, make

device names valid as files with BDOS doing the stream to block

conversion.

Several BDOS utility functions are proposed in order to make

life easier for the programmer.

1. Parse FCB.  Parses the next token in the command line into

a FCB.  This includes named directory resolution, password

protection, etc.

2. Full screen STDOUT string I/O functions.

3. AUX: status (used for modem)

Recommended BIOS revisions are:

1. Full track buffering.

2. Disk caching (write thru)

In addition, there may be other SYSLIB functions such as

those that get system addresses that may be desirable to

implement as BDOS/BIOS calls.

IV. MULTITASKING

Multitasking will require BDOS and BIOS to be-reenterant (or

alternatively, BIOS can be maintained as non-reenterant if a task

switch is disallowed until the IO operation is completed).  BDOS

must have a separate RAM area for each task (which must include a

separate DIR buffer).  Clearly, common memory can't be used for

this.  The data could be located in the system bank, but a better

solution would be to limit multitasking to the 180 and 280

processors, where changing the memory partitions on the fly is

simple.  This fits well with the current location of RAM in

ZSDOS.  BIOS variables could be stored in a similar matter.

TPA bank           Alt bank

bios bank point   +-----------------+ +---------------+

|     bdos        | | bios bank ram |

bdos bank point   +-----------------+ +---------------+

|                 | | bdos bank ram |

normal bank point   +-----------------+ +---------------+

Due to possible severe compatibility problems, I recommend

that multitasking be implemented only after the banked OS is

defined and implemented.  This suggestion does not mean that we

shouldn't carefully consider the impact of multitasking on the

banked standards!

V. SUMMARY

This represents some of my thoughts on banking.  I don't

make any claims to have all the answers - indeed my motivation to

write this document is to spur others to think, question, ponder,

and suggest improvements to the ideas presented here.  We don't

have anybody like Microsoft or IBM to drag us along.  This means

that for better or worse, we control our own destiny.  Let's get

together and discuss the issues banking raises so together we can

define the next generation of ZSystem.

I can be reached on the Ladera Z-Node (Al Hawley's wonderful

board) and on Z Node Central.  Leave a message for Cameron W.

Cotrill.

#######################################################################################