If you are lucky enough to have a VERSAFLOPPY II disk controller board you

will appreciate how reliable it is.  There was one problem however SD Systems

supplied a bios for their SDOS.	I wanted to run CP/M 2.2 and simmilar systems

The 5 programs:

MYDDBIOS.Z80

MYBOOT.Z80

DDSKBIOS.Z80

SYSGEN.Z80

FORMAT.Z80

should allow you to get CP/M running on this board.

Here is how it works.	First only two drives are assumed (A: & B:)  both must

be 8" drives. This saves a lot of extra code I did not want but others could

probably modify acordingly.	The major problem is the bios must know if it

is talking to a single density disk or double density disk (sd dd).	RAM

locations 58h & 59h will always contain flags for the density of drives A & B

Upon cold or warm boot the bios gets the density of disk A (see below). If it

is sd then 58h=00h if dd then 58h=40h.	Since drive B: has not yet been called

in a flag of 0ffh is put at 59h.	The first time B: drive is selected

its density is selected and the 0ffh flag is changed to either 00h or 40h.

When CP/M does a SELDSK it will be routed to one of two different disk

parameter lookup tables depending on the two above flags.

How does the board find out the density of the disk?	SD systems used a neet

trick.	They use a trial and error method.	Upon any read address command

the 4th byte returned is the sector length.	For 128 byte sectors it is

00h.	So if the read address command returns anything othere than 00h chances

are we do not have the correct disk density.	Look at my UNITSL: routine and

this will hopefully be clear.

I have made one large extra addition to the bios at the end you may want to

remove.	It is a section to display on a VDM board in "real time" the actual

sector,track,disk currently being read or written to. Bit 7 of port 0ffh turns

this info on or off.	It is great for debugging at the start.

All of the above and 99% of the bios is stored in ROM (just like SD SYSTEMS

do for their system) -- this is by the way MYDDBIOS.Z80

Two othere things are required:-

MYBOOT.Z80	is the CP/M boot loader to go with this system. As usuall it

goes on track 0 sector 1 of all disks.	This loader brings in the rest of CP/M

loads it at the correct place in RAM (this depends on the size of your CP/M

system) along with a tiny RAM based BIOS which I call:-

DDSKBIOS.Z80	As far as CP/M is conserned this is THE bios. It contains

all the appropiate jumps to the ROM bios and the RAM dependent disk parameter

lookup tables.

If you are new at this stuff get somebody to show you how to link in these

two programs to CP/M before sysgening a system.

The following is an example of a 60k system setup

ZSID

F100,4000,0

ICPM60.COM

R

ISYSGEN.COM		;I put sysgen here so I can have a file with sysgen

R			;and CP/M @ 900h saved (if this is not clear do it

IMYBOOT.HEX		;the way CP/M suggests)

R880

IDDSKBIOS.HEX

R3980

^C

SAVE 34 SYSGEN60.COM	;(This way I have sysgen + the CP/M image in memory)

SYSGEN60		;You are on your own after this

Good Luck

John Monahan