;**************************************************************

;

;			EPROM.DOC

;

;		LAST MODIFIED 9/27/80

;

;	EPROM PROGRAMMER SOFTWARE FOR RUNNING AN SSM - PB1

;	2708/2716 PROGRAMMER WITH CP/M UNDER DDT

;

;	TO RUN THIS PROGRAM, TYPE:

;

;DDT EPROM.COM(CR)

;

;	THE CONSOLE WILL RESPOND WITH:

;

;DDT VERS #.#

;NEXT  PC

;#### 0100

;-

;

;	INSERT A HEX FILE TO BE PLACED IN EPROM BY ENTERING:

;

;IFILENAME.HEX(CR)

;R[BIAS](CR)

;

;   (NOTE THAT BIAS WILL ALLOW THE ENTERED PROGRAM TO RESIDE

;   WITHIN 103H TO 'START' AS REQUIRED EVEN THOUGH THE PROGRAM

;   WAS ASSEMBLED FOR EXECUTION OUTSIDE THIS RANGE)

;

;   TO START THE PROGRAM FROM DDT, ENTER EITHER 'G(START:)'

;   OR 'G100'.  THE PROGRAM WILL ASK FOR EPROM TYPE SO AS

;   TO SET THE PROPER SIZE (1K FOR 2708 AND 2K FOR 2716/2516)

;   AND PROGRAMMING ROUTINE.

;

;   SUPPORTED FUNCTIONS ARE COPY, PROGRAM AND VERIFY WHICH ARE

;   SELF EXPLANATORY.  THE AUTOMATIC SEQUENCE FOR PROGRAMMING

;   AN EPROM IS TO FIRST CHECK FOR FULL ERASURE, THEN THE

;   PROGRAM CYCLE, FOLLOWED BY THE VERIFY ROUTINE.

;   (ANY FAILURE RESULTS IN AN ERROR MESSAGE AND A BELL

;   SO AS TO NOT GO UNNOTICED.)

;

;   AS CONTROL IS PASSED TO DDT AND YOU WISH TO

;   RETURN TO THE PROGRAMMER, JUST ENTER 'G' AGAIN.

;

;   EXIT FROM THE PROGRAM BY ENTERING A CONTROL C

;

;**************************************************************