DU-V75A.DOC

by Ward Christensen

with additional notes by Ron Fowler and Keith Petersen

(revised 7/23/81)

and still further revision by H.M. Van Tassell (3/6/84)

DU-75A.A86 is a CP/M-86 translation of the CP/M-80 version 7.5 with

added disk routines which allow operation under CP/M-86 ver 1.1 and 3.1

and CCP/M-86 version 3.1.  (note: CP/M-86 ver 3.1 has not been released

by Digital Research as of this date)

This program checks the CP/M version number and will not run unless

the operating system is one mentioned above.  However, with only minor

changes, the program can be made to run under MP/M-86 or the original

version of Concurrent CP/M.  Note that CCP/M-86 ver 3.1 replaces both

the MP/M-86 and original Concurrent CP/M operating systems.

The starting translation came from the SLICER USER GROUP

disk #2, via Micro Cornucopia, PO Box 223, Bend, OR

It did not indicate the name of the translator who should

be thanked for his work.  That version appeared as DU-75.A86

See original CP/M-80 version 7.5 for history of changes and full

credits for all contributing authors.

With the exception of my changing the multiple command symbol from ";"

to "!", which is the CP/M convention for multiple command symbol, the

original documentation still applies and is repeated here for easy

reference.

HOW TO USE DU ver 7.5-A DISK UTILITY

USE:

Any valid command string may be placed as an

operand of the original DU command, i.e.:

A>DU G0!D!G2!=OK<D><A><1A>!D  (NOTE: if your operating system allows

multiple commands, don't use "!" in

the DU command tail)

Functions supported:

Tnn	Seek to track nn (no read)

Snn	Position to sector nn, and read

Gnn	Position to group nn and read.

G	Shows current position

V	Views the current sector.

(assumes ASCII data)

Vnn	Views nn sectors

Fname	print directory for file "name",

and gives group information for

the first extant

=string	Ascii search, starting at current

sector. <xx> hex may be imbedded,

or used alone:  To find "IN 0FEH":

=<db><fe>

Ignores bit 7 unless <xx> is used.

L	Re-logs in the current disk. You may pull

out a disk, put in a new, and "L" just

to log it in.

Lx	Logs in disk 'x', such as: LB

Ux	Logs user 'x' for next F command.  Gives

'?' error if not CP/M version 2.x.

D	Dump sector, hex + ASCII

A	Dump sector, ASCII only

H	Dump sector, hex only

note all dump commands (D, A, H) may be optionally

followed by a starting and ending address:

D0,7F	is the same as just D

D3,5

A20,3F

CHaddr,val,val,val... change hex in sector

CAaddr,char string... change ASCII in sector

NOTE that <xx> may be hex imbedded

in the Ascii:  ca0,OK<d><a><1a>

----> Use W to write changes to disk.

Note that the C command echoes

the overlaid data for verification.

CHaddr-addr,byte

or	CAaddr-addr,byte	repeats a change

+	advance 1 sector (if below track 2,

this advances to next numerical, if

2 or >, advances based on CP/M's normal

sector scrambling algorithm, i.e. so +

will get the next logical sector of the file

-	backs up 1 logical sector

Note + and - may take an amount:

for example, +F steps in 15 sectors.

Note also that "-" issued at the first logical

sector of the disk will wrap back to the last.

Further, "+" issued at the last sector will

wrap forward to the first.

?	Gives command summary

#	Prints the disk parameters

M	Dumps a map of the group allocations

for files.

Mn	Shows which file is allocated to

group "n".

N	Resets CP/M via the BDOS.  This may

make it possible under some implementations

of CP/M to change the disk format (e.g., density,

sides, etc)

R	Reads the sector currently positioned to

into memory.  Note R (Read) is implicit in

the G, +, and - commands, but N-O-T in the

S and T commands (I did it because I was

tired of disk reading after T command before

I had a chance to issue the S command)

W	Write back the current sector (N-O-T-E may

not be used after an F command, as CP/M was

used to find the file in the directory

X	Exit back to CP/M (Must press return).  Ctl-c

was too easy to hit over modem lines, so I

decided on 2-byte (X, CR) to exit.

P	Toggle printer switch on/off

Y       Yanks current sector into sequential memory and

prints ending address.  This allows sectors to

be written sequentially into memory starting at

3000H, for later recovery by DDT, etc.

Z	Sleep - causes the program to pause, such

as to look at a dump.  Z is 1 sec.  Znn

is nn tenths of a second, more or less.

<	Saves current sector in a save buffer

>	Gets saved buffer.  < and > may be used

to move a sector to another place.

/	Repeats entire command.  Defaults

or	/nn	to "forever".  NN may be 2 to 65535

multiple commands may be separated by "!"

Example: the following commands will erase the

b disk directory to all E5's:

lb		log in b drive

g0		position to dir.

ch0-7f,e5	fill with e5

<		save the sector

>!w!+!/16	restore, write, next,

repeat 16

----This could be shortened to:

lb!g0!ch0-7f,e5!<

>!w!+!/16

INTERPRETATION OF DIRECTORY DATA PRESENTED BY THE DU PROGRAM

The following explains the format of a CP/M directory entry

as shown by DU, using either the "F" (find file) command,

or just doing "D" (dump) of the directory sectors, which

are located in groups 0 and 1 on a single density disk.

----------------

Sample result of "FSID.COM" command:

40  00534944 20202020  20434F4D 0000003A  *.SID     COM...:*

50  33343536 3738393A  00000000 00000000  *3456789:........*

First line -

40  00534944 20202020  20434F4D 0000003A  *.SID     COM...:*

||  |||                      |  ||    ||    |         |

||  ||^----hex file name-----^  ||    ||    ^file name^

||  ||                          ||    ||     in ASCII

||  ||                   extent-^^    ||

||  ||                                ||

||  ||           file size in sectors-^^

||  ||

||  ^^-00 = file active

||     E5 = file erased

^^-displacement of line in directory sector

Second line -

50  33343536 3738393A  00000000 00000000  *3456789:........*

|				       |

|				       |

^---allocation group numbers-------^

END