DOCUMENTATION FOR BACKMW.COM v2.50

BACKMW is (c) S. Kluger, 1984. The object code has been released into

the Public Domain and may be distributed freely, except for profit.

1. WHAT IS BACKMW

BACKMW is an incremental backup/recovery utility written for Morrow's

M16 hard disk system (hence the MW in BACKMW). It can be used on any

hard disk system provided that:

- the DPB of the highest logical drive includes all lower

logical drives as RESERVED TRACKS

- the hard disk does not have more than 1000 cylinders

BACKMW is used once to toally back up the complete hard disk onto

floppies, then it can be used periodically to update the information

backed up. It automatically keeps track of CRC values for all cylinders

and will back up only those cylinders that have been written to since the

last backup. While BACKMW can be used for partial recovery, writing back

to the hard disk only those cylinders that have changed since the last backup,

it is really intended for catastrophic failures, such as trashed logical

drives or a complete reformat.

2. OPERATION

BACKMW has four options, which are displayed in the main menu when

the program is started. Those options are:

T - total hard disk backup

I - incremental hard disk backup or restore

C - configuration parameter editor

Q - quit to CP/M

Simply select one of the above choices. Note that you must do a total

backup of your hard disk before you can do any incremental backups or

restores.

3. THE CONFIGURATION EDITOR

The editor is presented first, because your system may be different from

the default and you should first configure BACKMW for your environment.

Selecting the C command first displays the current configuration parameters

which default to my system environment:

High hard drive       : E:

Floppy backup drive   : F:

CRC file drive        : F:

Number of CYLINDERS   : 306       (306/432 is the data for the

Number of SECTORS/CYL : 432        Morrow M16 system)

CHANGE? (H,F,C,T,S,Q,?)

Simply type the appropriate letter to change an option (a "?" explains

the option letters) or use "Q" to exit the editor. After exiting, you may

optionally save BACKMW to a new file (you get to specify drive and file name).

4. TOTAL BACKUP

After selecting "T", you will be asked to input the size of your floppy

disks in k bytes. Be sure to enter an accurate value and that all floppy

disks you will be using during the backup process have AT LEAST that

much room left! You will receive a caution message if you select a floppy

size of less than 256k, but may go ahead with the operation anyway. Next,

BACKMW will tell you how many diskettes you need to back up the complete

hard disk. You should make sure you have enough floppies. While you can

abort BACKMW at any prompt (and, therefore, also while backing up), it is

not advisable to abort BACKMW in the middle of a backup operation, since

you cannot continue where you left off at a later date - you would HAVE TO

restart backup from scratch.

Next, you will be asked for an 8-character DISK ID, which can be almost

anything, but you might want to make it the current date. This 8-character

string will be used by BACKMW to construct the filename for the cylinder

files and the CRC file. The name MUST be 8 characters exactly.

You will now be prompted to insert the first floppy diskette and backup

will commence. During backup, the following information will be displayed

for each cylinder:

- cylinder number

- sector being read (updated continuously while reading)

- CRC value for the current cylinder

After BACKMW has determined that no more cylinders will fit on the current

floppy, you will be prompted to insert the next floppy and the cycle will

continue until all cylinders have been backed up. At 432 sectors per

cylinder, each cylinder takes approximately 30 seconds, so an M16 hard

disk can be backed up in about 2 1/2 hours.

When BACKMW has finished the backup operation, it will prompt you to insert

the floppy on which the CRC file should be written. IF THERE IS SUFFICIENT

ROOM, you should write this file to the very first disk backed up. Otherwise,

send it to a disk not used in the backup process. The CRC file contains

8 bytes for each cylinder, so in the case of an M16, you must have at least

4k to store the file! Do not write the CRC file to the hard disk, for

obvious reasons!

5. INCREMENTAL BACKUP / RECOVERY

If you select "I" at the main menu, you will be asked whether you want a

BACKUP or RECOVERY operation. Backup means FROM HARD DISK TO FLOPPY,

and recovery means FROM FLOPPY TO HARD DISK. Any response other than "B"

or "R" will return you to the main menu.

BACKMW will immediately ask you to insert the floppy which contains the

CRC file, and it will read the CRC file into memory. Next, you will be asked

for the starting cylinder number. This allows you to do partial backup/restore

or continue a previously aborted incremental B/R. The operation you selected

(B/R) will begin immediately. BACKMW CRC-checks each cylinder on the hard

disk, displaying status information as it goes. If the CRC calculated in not

the same as the value in the CRC file, BACKMW will attempt to open the

appropriate cylinder file on the floppy and transfer the information in that

file back onto the hard disk if RESTORE was selected, or it will update the

cylinder file on the floppy disk if BACKUP was selected. If BACKMW cannot

find the cylinder file, it will prompt you to insert the appropriate disk

(by number) until it finds the file.

After finishing an incremental BACKUP, BACKMW writes the CRC file back

onto the disk it read it from. No CRC file is written after a RESTORE

operation.

CONTROL-C can be used to abort the incremental B/R operation at any time.

When aborting a BACKUP, be sure to allow BACKMW to write the CRC file!!!

BACKMW will want to write the CRC file after an aborted RESTORE too, but

you may abort this before writing.

6. EXIT

BACKMW performs a warmboot when executing the Q command.

7. ERRORS

BACKMW will abort to the main menu any time a READ, WRITE, or OPEN error

is encounteded. The filename which caused the error will be displayed.

Should an error be encountered during hard disk write, a BIOS ERR R/W message

will be displayed and BACKMW will execute a warm boot.

8. MISCELLANEOUS

I have tested BACKMW on an M16 system. While testing, I inadvertently

clobbered my directory on A: and I was able to use BACKMW to restore it,

which proved to me that it worked. Even though I have used the program

many times and it appears to work fine, please use it with caution.

Be sure to do a conventional backup before using it for the

first time. Ideally, the following should be done to test the program, but

since I run a RCPM I don't have the time to do it:

1. do total backup

2. run a whole disk checksum (like the TEST command in POWER)

3. clobber about 50% of the data on the drive

4. do an incremental restore

5. verify that whole disk checksum produces the same

result as in (2) above.

If you setect any bugs, failures, strangenesses, please let me know with an

exact description of the problem. Verify that the problem is not random.

9. CREDITS

Thanks to frank Wancho for suggesting the incremental backup method

Thanks to Rick Conn for SYSLIB

Thanks to whoever uses it and reports success/failure

Dallas, TX, 06-24-84

ADDITIONAL NOTES: (this used to be BACKMW.NOT)

For more info read BACKMW.DOC

(updated 6/24/84)

TOTAL hard disk backup should ideally be done only once, unless more

than 40% of the hard disk has been changed. Since total foolproofing

of BACKMW would have added to the complexity and size of the program,

I have decided not to incorporate any BDOS error trapping. This means

that you MUST be absolutely sure that when you do a total backup,

1. All floppy disks you are about to use are EMPTY, freshly formatted

2. All floppies have the same capacity and you specify that capacity

when BACKMW2 asks for it

3. You never make any silly mistakes such as leaving the drive door

open, reversing the disk etc,

for if you do you will be forced to restart from scratch! On an M16

system that means you have to spend another 2 1/2 hours, PLUS the time

it takes to ERA *.* all diskettes again!

Now, now... don't let those little problems keep you away from a

free program...

VER 2.2:

A number of bug fixes in the incremental routine, which caused the system

to hang, access the floppy at the wrong time and mess up the CRC file.

A ^C abort feature was added to the incremental routines, where the CRC

check of the current cylinder can be aborted at any time. When aborted,

you will be asked to insert the CRC file diskette and the CRC file will

be updated in accordance with your actions.

VER 2.3:

V2.3 now runs on banked CP/M plus systems. BIOS calls are made through

BDOS call 50 IAW the CP/M plus calling conventions.

VER 2.5:

BACKMW version 2.5 no longer supports the buggy "manual" restore mode

introduced in v2.4. When doing an incremental backup or restore, you

are now asked for a starting cylinder number. This way you can interrupt

your work at any point and continue later without CRC checking hundreds

of already-processed cylinders.

This version will not insist on writing the CRC file if you run a

RESTORE to completion. Aborting an incremental function with ^C will

still result in the CRC file prompt. BE SURE to write the CRC file

after aborting an incremental BACKUP!