Howdy..feeling lousy, could not sleep, head splitting, nose plugged, ears

snappy..so I tinkered around..now is 4:15 am...been studying disk drives, call-

ing BBs and found below at CC BB and downloaded that. Studied the TEAC manual

to find out why Elephant disk did not work and think I found out.  The drives

operate on two methods, MF and MFM, whatever they mean. MF is lower or 40 tracks

and MFM is higher or 80 tracks. I compared them with TEAC55B and TEAC55F and

came upon interesting information.

On B, Capacity (Kbytes), FM=163.84, MFM=327.68 and on F, FM=327.68, MFM=655.36.

You can see that 163.84 has shifted from MF to MFM in F. Looks like the two

drives are very different animals and can't talk to each other. With this info,

I am going to try experement with your TEAC45B and my drives and see If I can

arrive at something. Failing this, it means we wont be able to exchange disks

unless we config a disk to KAYPRO II and copy from those.   Cheerios Motty

Drive Jumpering for the SB180

Tandon 65-2 and Canon 211 drives don't work on the SB180.

5.25" 48tpi drives:

TEAC FD-55B-20-U		    : ML,UR,DSx

TEAC FD-55B-01-U		    : HM,SM,UR,PM,DSx

TEAC FD-55B-01-U (other one)  : HM,ST,DSx

TEAC FD-55BV-06-U 	    : HL,RY,DSx

TEAC FD-55BV-36-U 	    : HL,RY,DSx

Matsushita JA-551-3	    : DSx,DS,+WP,MM,IU

Matsushita JA-551-045	    : DSx,DS,WP,MM,DR,RD,RY

Shugart SA450		    : DSx,DS,MM,IU

Shugart SA455-3CAE	    : DSx,DS,+WP,MM,IU

Mitsubishi M4851-112U	    : DSx,2S,MM,HM

5.25" 96tpi drives:

TEAC FD-55F-03-U		    : HM,SM,UR,PM,DSx

8" drives:

Shugart SA851		    : DSx,850,2S,Z,A,B,I,R,IW,

S2,IT,C,RS,HLL,M,NF

The following configurations were submitted by SB180 users

and have not been verified by Micromint:

5.25" 48tpi drives:

Qume 542			    : HM,DSx,HL

5.25" 96tpi drives:

Mitsubishi M4853		    : HM,MM,H2,R3,DSx

8" drives:

Shugart SA801		    : DSx,800,A,B,C,Z,DS,L

Shugart 860-1		    : DSx,MS,Z,MD,SR,S2,2S,B,A

Qume 842			    : DSx,2S,A,B,C,D,Y,R,I

Notes:

1) The DSx entry used throughout signifies the appropriate drive

select option for the location of the drive in your system.

Most drives have either DS0,DS1,DS2,DS3 or DS1,DS2,DS3,DS4.

2) The terminator pack, if present, should be installed on the

drive physically at the end of the cable only.	It should be

removed from all other drives on the system.