SECTION 2.0 GETTING STARTED

Before you can start using Flipper, you need to set up one of the

supported synthesizers, and then tell Flipper which kind you are

using.  In most cases, this will consist of setting up the

synthesizer, and then running Flipper's INSTALL program to

set up the batch file that you need to load Flipper.  If you prefer

to write the batch file yourself, you should have INSTALL

write an example batch file and then modify it or use its contents

to write your own batch file.  A plain text editor, called

EDDIE, is included with Flipper and is useful for writing and

modifying these batch files.

The instructions for installing the internal cards are included

with the synthesizers, but we suggest that you either have your

computer store or an experienced friend do the installation if you

have not installed cards before.  The installation, however, is

quite straightforward.  Internal cards require software to be

loaded, either in a CONFIG.SYS file or in a batch file, as

explained in their instructions.

The INSTALL program which is included on the distribution

disk will automatically configure Flipper for use on your

computer. Your computer should be set up to boot MS-DOS

before installing Flipper.  To run INSTALL, put the installation

disk into your floppy drive and type INSTALL followed by a

return.  Follow the instructions, and Flipper will be installed on

your system.

There are now separate Voice Modules for the various

synthesizers.  This allows Flipper to optimize its commands for

the synthesizer you are using, taking advantage of many

synthesizers' advanced features.  Only the information for your

particular synthesizer is loaded into memory for Flipper's use.

Instead of specifying the synthesizer you are using when you run

Flipper, itself (as in flipper m2 for the Accent SA on com port

2), you now run a Flipper Voice Module before running

FLIPPER.COM. The voice modules begin with F_ and are

.COM files.

SPECIAL INSTRUCTIONS FOR SERIAL SYNTHESIZER

INSTALLATION:

Serial synthesizers need a working two way connection.  If

Flipper can communicate with the synthesizer, but the

synthesizer can not send characters back, Flipper will only speak

for a fragment of a sentence at a time, and then wait for the

synthesizer.  This can be caused by a bad serial cable, a bad

serial card, or more commonly, by two or more serial cards or

other devices set to the same interrupt or port number.

DECTALK:

The Dectalk should be connected using the cable which is

included with the Dectalk.  The cable should be connected to the

port labeled COM on the back of the Dectalk.  The factory

setting for the Dectalk is 1200 Baud, and if it is not changed,

you should tell the install program to set up for that speed.

Setting the Dectalk to 9600 baud will not significantly increase

its response speed.

THE ACCENT SA:

The Accent SA uses a straight through cable, and used at its

factory default settings, which are 9600 baud, even parity, one

stop bit, 7 bits per character and software (Xon, Xoff)

handshaking. Flipper can be set to use the Accent SA on COM3

or COM4 without an interrupt, if they are supported by your

BIOS. On PC and AT compatibles, this will usually require

separate software.

THE APOLLO:

The Apollo should be set to its factory default, which is 9600

baud, one stop bit, no parity, 8 bit characters, and it should be

used with the supplied serial cable.

THE AUDAPTER:

The Audapter should be set to its factory default, which is 9600

baud, one stop bit, no parity, 7 bit characters, and it should be

used with the supplied serial cable.  Flipper can be set to use the

Audapter on COM3 or COM4 without an interrupt, if they are

supported by your BIOS.  On PC and AT compatibles, this will

usually require separate software.

THE LIGHTALK AND DOUBLETALK LT:

The LIGHTALK and DOUBLETALK LT should be set to their

factory default, which is 9600 baud, one stop bit, no parity, 8

bit characters, and they should be used with the supplied serial

cable.  The LIGHTALK can alternatively be connected to a

parallel port.  The voice module for the doubletalk synthesizers

is F_double.

OTHER SERIAL SYNTHESIZERS:

In general, Flipper can be used with the serial cables included

with these voice products.  The only connection the PC needs is

a three wire, null modem connection.  However, several of the

synthesizers need handshaking wires which are included in their

factory supplied cables.  These synthesizers should be set to

9600 baud, even parity, one stop bit, 7 bits per character and

software (Xon, Xoff) handshaking.

You must tell Flipper where to find the synthesizer by using the

following command line arguments after the Voice Module: S1

for a serial synthesizer on com port 1;  S2 for a serial

synthesizer on com port 2.

To run Flipper with an Accent SA on Com 2, for example, you

would use the commands:

F_ACCENT S2

FLIPPER

When the Voice Module is loaded, you should hear Voice

Ready. Then, when Flipper, itself, is loaded, you should hear

the Flipper ready message.

Note: It is still necessary to run drivers for those synthesizers

which require them.  For example, you must still run

SONIXTTS before you run F_ARTIC.COM AND

FLIPPER.COM. To load Flipper 4.0 for use with an Artic 210,

for example, you would use the following commands:

SONIXTTS

F_ARTIC

FLIPPER

Options which the install program can add to the command line

for Flipper itself include:

Braille to parallel device                            =<parallel port>

Braille to serial device                              +<serial port>

Do not load built in help                             h

Do not load built in documentation                    d

use a review mode cursor that is trackable            c

block a hardware interrupt                            x<interrupt>

change the size of the grab buffer                    g<size in bytes>

The items in angle brackets are numbers, single digits except for

the size of the grab buffer.

The Voice Modules are as follows:

driver            Synthesizer support:

F_ACCENT          Accent for the Toshiba

Accent PC

Accent SA

F_DOUBLE          doubletalk

doubletalk_lt

F_DOUBLE          litetalker

F_ARTIC           210

Artic 220 /S2

Keynote PC or Keynote PC+

Artic 200

Artic 220

Votrax Votalker

F_MORSE           PC speaker morse code

F_DEC_PC          Dectalk PC

F_AUDAPT          Audapter from Personal Data Systems

F_DEC_C           Dectalk

Dectalk

Kursweil

Speakeasy

F_MULTI           multivoice

F_SBLAST          sound blaster

F_ECHO            Echo GP or PC

F_ECHO_N          New echo

F_B_N_S           Braille and Speak

F_SB              Sounding Board

other porttalk synthesizer

F_P_B             other echo subset serial synthesizer

Pocket Braille

F_APOLLO          Apollo

F_VOTRAX          Votrax PSS

F_ASP             Asp