The other files in this ZIP (PSLWGRAY.PSF, PSLWRGB.PSF, and

PSLWCMYK.PSF) contain version 1.4 of some PostScript code that allows

control over minimum line width and output color or gray level, on a

color-by-color basis, for PostScript output using the AutoCAD Release

12 PSOUT command.  The code has been tested on an NEC Silentwriter

90, an Agfa 2400 dpi PostScript printer, the Ghostscript interpreter

version 2.61, and Freedom of Press Classic 4.0 printing to an HP

Deskjet 500c.  Using the code in this package requires AutoCAD

Release 12 (or higher).

All three files define a minimum line width in inches for each

AutoCAD color. Entities with explicit widths (for example, plines)

that are greater than the minimum for that color will be printed with

their explicit width (this assumes interpreting AutoCAD drawing units

as inches).  In rare cases some lines may be wider than desired (when

the AutoCAD PSOUT command temporarily makes the X and Y axes have

unequal scales).

Each file defines a "printed" color for each AutoCAD color, but

each file does it in a different way and with different capabilities

and limitations.  PSLWGRAY produces monochrome output, black or white

or shades of gray.  PSLWRGB produces color output, specifying colors

as combinations of the primaries RGB (Red-Green-Blue).  PSLWCMYK

produces color output, specifying colors as combinations of the

primaries CMYK (Cyan-Magenta-Yellow-blacK).  Note that the CMYK color

model is a PostScript level 2 feature, and may or may not be

supported by any particular PostScript level 1 device.

Due to limitations in the way AutoCAD handles the PSOUT command

and the PSPROLOG system variable, it's not particulary convenient to

change line widths and colors or gray scales.  I hope this will

improve in the future.  For now, you must edit either ACAD.PSF or

each individual .EPS file produced by PSOUT to change the line widths

or colors or gray scales.  Changes in ACAD.PSF affect all files

produced by PSOUT after the change; changes in an .EPS file produced

by PSOUT affect only that file.

Currently, PSLWGS only supports portrait output.  This means

that lines that are vertical on the AutoCAD screen will be parallel

to the long edge of the paper when printed.  If you're doing PSOUT to

import to another program and need another orientation, the program

into which you're importing the .EPS file may be able to rotate it.

To use this file:

I.  Initial setup:

A.  Locate the file ACAD.PSF.  It's probably in the SUPPORT

subdirectory of your AutoCAD directory.

B.  Use an ASCII text editor to insert any (or all) of the three

.PSF files in this package, at the end of ACAD.PSF.  (ACAD.PSF

is covered in the Release 12 "Customization Manual" on pages

228-232).

C.  If you will normally use line widths other than 0.001" (note

that a 300 dpi printer does line widths in units of 0.0033"),

change the values in LGP_colortowidth (see step C, below) to

be your most common linewidth settings.

II.  Using one of the prologs

A.  In the AutoCAD drawing editor, set the PSPROLOG system

variable to contain "ps_lw_gs" (for gray scale output) or

"ps_lw_rgb" (for RGB output) or "ps_lw_cmyk" (for CMYK output).

NOTE THAT THIS IS CASE SENSITIVE!  For example, you can set up

for gray scale output by typing:

psprolog ps_lw_gs<Enter>

(Note that PSPROLOG is saved in ACAD.CFG and not with the

drawing.  This means that this setting will apply to all

your drawings until you change it).

B.  Perform a PSOUT command.

C.  If you want to change the line widths from the defaults

(0.001" if you haven't changed them), use an ASCII text

editor to edit the resulting .EPS file. Near the beginning

of the file you'll find:

/LGP_colortowidth

[

0.001          % Color 1 (red)

0.001          % Color 2 (yellow)

0.001          % Color 3 (green)

0.001          % Color 4 (cyan)

0.001          % Color 5 (blue)

0.001          % Color 6 (magenta)

0.001          % Color 7 (white)

0.001          % Color 8

0.001          % Color 9

0.001          % Color 10

0.001          % Color 11

0.001          % Color 12

0.001          % Color 13

0.001          % Color 14

0.001          % Color 15

] def

Change the numbers as desired.  The units are inches. Note

that on a 300 dpi printer, the minimum line width is 0.003333

inches; any specified line width that's less than that will

come out as 0.003333 inches.

D.  If you want to change the colors or gray scales from the defaults

(all black for monochrome output, or the same as the standard

AutoCAD color [in 16 color mode] for color output), look just

below the line width section.

You'll find some comments (indicated by a "%" sign) explaining

the meaning of the numbers for the particular color model being

used, and the definition of the mapping array.

Change the numbers in the "LGP_ACADcolortoPScolor" array as

appropriate, and as explained in the comments preceding the

array.

E.  Save the file and copy it to your output device.

Jon Fleming

CIS:     70334,2443

REVISION HISTORY:

1.0 Initial release

1.1 Fixed bug of leaving an object on the stack after the entire

procedure finished

1.2 (13 Jul 93) Fixed bug of desired line width not saved and

restored with graphics state.  This manifested as "persistent

polyline widths"; when a polyline with explicit width greater

than the minimum was printed, all subsequent strokes were at the

same width as the polyline until a color change or another

polyline came along.

1.3 (11 Oct 93)  Added RGB and CMYK color capability.  Note that the

three files are identical except in the definitions of the

"LGP_ACADcolortoPScolor" array and the "LGP_setcolor" procedure.

Added semicolons to the front of most comments and blank lines

so they won't show up in the final EPS file.

1.4 (20 Aug 94)  Added the UserPrologDict private dictionary and the

LGP_Version entry in that dictionary.  This put PSLWGS into better

conformance with reccomendations for EPS files, and (for some

reason I don't understand) allows Autodesk's ALlogo PSFILL to work

in a file that conatins a PSLWGS prolog.