NB have edited to the end of Macro 13-8-94

***********************************************************************

*                                                                     *

*              *      *   *        ******  *           *              *

*              *      *   *        *        *    *    *               *

*               *    *    *        ***       *   *   *                *

*                *  *     *        *          * * * *                 *

*                 **      *        ******      *   *                  *

*                                                                     *

***********************************************************************

A data analysis suite,

incorporating the exponential fitting routine DELP.

DELP algorithm due to Dr J Martin,  Dept  Physics,  Kings  College,

Strand,  LONDON WC2R 2LS,  and implemented by DJ Maconochie, Washington

University BOX  8054,  660  S  Euclid,  St  Louis  MO  63110  (to  whom

correspondence may be initially addressed.

E-mail DJM@morpheus.wustl.edu

djm16@leicester.ac.uk (Fall 1994)

Feel  free  to  use  this  program,  but  please  reference  Martin,

Maconochie and Knight (1994),  J Neurosci Meth Jan 1994 V51 P135-146 in

any publications that contain data analysed using DELP.

CONTENTS

1) INTRODUCTION

2) SYSTEM REQUIREMENTS

3) BRIEF OVERVIEW

4) GETTING STARTED QUICKLY

5) MENU TREE IN DETAIL

6) SPECIFIC TASKS

_____________________________________________________________________

1)                         INTRODUCTION

This  software  comes  free,  with  no  guarantees.  Feel  free  to

distribute it in its entirety, with all the associated files.

The following features are available

*) averaging, with automatic and manual alignment

*) very fast multi-exponential fitting

*) laser printer output with some editing facilities

*) multiple display and plotting windows

*) contents of file and files in a directory plotted automatically

*) automatic trace by trace subtraction for

i) response-control

ii) transient and leak subtraction

*) calculation of mean, minimum and maximum values

*) filters: analogue, non-causal, Savitsky Golay, Wiener

*) digital differentiation and integration

*) Fast Fourier Transforms, power spectra with averaging and smoothing

*) Editing of data files

*) Detection of events: single channel, synaptic etc

*) Ohmic series resistance compensation

*) Transformation by log, power, multiply

*) clampex, sigav and generic files types supported (up to a point)

*) data arrays of up to 32,000 points

*) monitor LANDSCAPE and PORTRAIT modes

____________________________________________________________________________

2) System requirements:

a)  Processor-  Any  IBM  PC  or  compatible should be able to run this

program, the program also appears to run on Sun workstations with a DOS

window.

b) Maths co-processor- By default,  the code is compiled for use with a

maths  co-processor.  There  is  no  reason for this except that I have

always had one, and without one the program will be slow.  For example,

filtering the data digitally might take 4  seconds  on  a  33  MHz  486

machine, and 20 minutes on a 286 machine without a co-processor.

c)  Screen-  The suite was written with standard 16 colour VGA in mind.

It should run on any screen,  although there  may  be  difficulties  in

running extended VGA modes on cards that do not support them!

d) Printer- Hard copy was available originally on a dot matrix printer.

Most dot matrix printers should  give  something  resembling  a  screen

dump.  Plots  are supported on HP�II,  HP�III and HP4.  You are however

limited to the fonts available (for example the HP II  only  has  "line

printer"  and  "courier").  HPGL output is also available:  it has been

used  successfully  on  a HP�III and 4 printers,  and incorporated into

VENTURA documents.

e) Data- The most important part.  Any file is acceptable that contains

a (optional) header string,  followed by a sequence of 2-byte integers.

The following extensions are reserved,  and cause the program to try to

read a particular type of header from the start of  the  file:  ".SVG",

".OLV", ".CX4" and "CX5".

f) Memory!  How much?  The absolute  minimum  at  present  required  to

operate  all  the features in standard VGA is 500k.  This is the number

that you should see when you type "mem" from a DOS prompt.  The program

works  much better if you have a EMS driver loaded and a modest 250k of

EMS memory free.  A DPMI version is also available,  for which you will

need  a  run  time  manager or RTM.  You are on your own with this one,

since I can only say that the program works fine in  a  protected  mode

emulation (OS2 2.1).

____________________________________________________________________________

3) Brief overview:

It  was  my  aim in writing this manual to use clear Englissh (with a

few typos!), and complete sentences.  I also wished to avoid the use of

such entertaining and philosophical but ambiguous statements as: "Space

continues!". No doubt, like life, love and war, it does.

The program runs entirely in graphics mode.  There is a central  menu

tree  root,  from which branches extent vertically in levels.  <Return>

and <Esc> move up and down the levels.  There is also a  Display  mode,

from which a large number of functions are available.

Brief  information/ instrucions are available in a "Help" line at the

bottom of the screen.  In addition,  from  the  "Display"  mode,  press

<Alt>+"H".

Items from the central menu tree may be selected in one of four ways:

a)  items  in the root may be selected from anywhere in the program by

typeing <Alt> plus the first letter.

b) items in any given sub-menu can be selected  by  typing  the  first

letter.

c) by moving the  highlighted  bar  using  the  arrow  keys,  pressing

<Return>.

d)  By  taking  one  of a number of shortcuts such as <Alt>+"R" to run

DELP,  <Alt>+"L" to load a file,  <Alt>+"X" to leave  the  program  and

<Alt>+"G"  to get the data display screen.  The shortcuts are indicated

to the right of the item in the menu tree.

So for example,  from anywhere  in  the  menu  tree,  either  of  the

following will load a fileto load a file you can:

a) type <Alt>+"F", followed by "L" for load, or

b) type <Alt>+"L"

_____________________________________________________________________

4)                     GETTING STARTED QUICKLY:

From the DOS prompt to an exponential fit in 9 easy steps!

It  is  assumed  that

a)  you  have  some  data  in  a  file  called  "filename.dat" or some

equivalent.

b) the data is a sequence of 2 byte integers.

c) that you wish to fit a sum of exponentials to the data

So do the following

i) from  the  DOS  prompt,  type  "VIEWMENU"  <Return>.

ii)  type <Return> three times.  After the second,  you enter the File

menu.  After the third,  you invoke the last data file that  was  used.

The filename may be edited.  Then press <Return> again.

ii) hold the <Alt> key and press "G" (ie type <Alt>+"G").  You are now

in the display mode.

iii) press the <Home> key.  This has the effect of displaying the data

on screen at minimum magnification.  If the trace  is  an  inconvenient

size,  then  read  the  following section to find out how to change the

size of the diplayed trace:  section Graph-Display- below.  If the data

is  very small,  then it may be in the lower 12 bits (for example if it

was collected using AXON software).  In this event, go back to the File

menu (<Alt>+"F") and press "1" to turn on the "12 to 16" bits option.

iv) press  <Return>.  Now  you  should  see  two  cursors  on  screen.

Position them around the section of data to be fitted, using <Ctrl> and

the arrow keys.  Press <Esc> when you are finished.

v)  type  <Alt>+"D".  You  are now in the fitting menu for DELP.  Type

"i" and enter the number of  exponentials  to  be  fitted  (Integration

level).  Type  "p"  and enter the polynomial degree to be used.  Ten is

usually sufficient, but experiment with more or less.

vi) If next to "Baseline" the word "SET" appears,  then press "B".  In

"AUTO" mode, the baseline or offset in your data is calculated with the

exponential rates.

vii)  now  press  "R" to begin the fit.  First a red line overlays the

data.  This is a curve calculated from the polynomial coefficients used

to represent the data.  Next the complete fit is drawn  in  dark  blue,

and  a  box with the calculated coefficients appears.  Each coefficient

appears as a complex number, for example "-0.5067 + 0.003j".  If a pair

of coefficients is complex,  then the data  has  been  fitted  with  an

oscillatory component (ie not a simple decaying exponential).

In  addtion,  in  a separate window,  you will see another noisy blue

trace (the residual) and an  even  wilder  red  trace  (the  integrated

residual.

viii)  press either "O" to overwrite,  or "A" to append the desciption

of the fit to an ASCII file.

ix) type <Alt>+"G" to return to the display mode, or <Alt>+"X" to exit

the program.

x) suppose that you had more than one sweep of data in the file.  The

program  may  have  set the number of points per sweep to either 32,000

(the maximum) or to some other number equal to the file size.  You  can

reset  the correct number of points per sweep by typing <Alt>+"T",  "R"

to get the "Transfrom-Rescale-" menu. Press "P" for "Pts per sweep" and

enter the correct number and press <Enter>.

Return to the Display mode by  pressing  <Alt>+"G",  and  then  press

<Home> to restore the default size.  and then either reload the file or

move to another sweep with <Tab> or <spacebar>.

_______________________________________________________________________

5)  MENU TREE IN DETAIL

In the following sections,  the functions available after  selecting

the indicate item are given.

___________________________________________________________________________

FILE MENU

File-Load- re-load the last file you used by  pressing  <Return>,  or

select  a  new  one  by typing the full path and name.  When you load a

file successfully,  if the file does not have a recognised header  that

gives  the digitisation frequency etc,  you will be asked "retain scale

values?  Y".  If you answer "Y", the last values used will be retained,

otherwise default values will be set.

If  the  name  you  type  is  not  valid  for  some reason,  you will

automatically enter a directory utility (see -Directory below).

File-Pick- pick one of the last six files that you used,  or  one  of

the standard files..

File-Change dir- type in a path to search.  Do this before trying the

directory search!

File-Directory- searches the given path and file template eg "*.*" or

"*.dat". It is a good idea to enter the directory path from File-Change

dir  first.  To begin the search,  type <Return>.  A box with a list of

file names should appear.  Directory names will be pre-fixed with  "\".

The  symbol "\.." in the top left corner indicates a route back towards

the root directory or drive.  If in doubt,  exit by typing  <Esc>,  and

giving the template "*.*", work your way out from the current directory

(where the program is,  usually "C:\VIEW").  The operation of this item

can be a little confusing.

File-Quit- Exit the program,  saving to disk all the parameters  that

have  been altered,  so that when you restart the program everything is

exactly as you left it.  The quickest way out is  <Alt>+X,  which  will

get you out from almost anywhere!

-BGI driver- toggles the graphics  driver.  There  are  a  number  of

alternative  grapics  drivers,  default  and VGA are only ones that are

guaranteed.  Default should include any BGI driver appropriate for your

screen type eg Hercules, Wyse etc.

-Mode- toggles the choice of modes for each graphics driver

-Reset- resets the screen mode following a change of driver or a  new

mode  selection.  If the screen goes blank,  then type <Alt+X> to exit,

from the DOS prompt type del '*.par',  and re-load the program.  If  an

invalid  mode  is  detected,  at  this point the parameter file will be

automatically reset to default, and the program will exit gracefully.

If none of the above work,  and you still start up with  a  blank  or

confused screen, then form a DOS prompt delete the file VIEW87.PAR, and

restart the program.

-12 to 16- when ON,  automatically multiplies incoming  data  by  2^4

(16).  Useful for loading data digitised with a lower 12-bit converter.

-Invert on loading- when ON,  inverts all incoming data, although the

data file is not changed.  The scaling is however unchanged.

-Save setup- every parameter in the program that may  be  altered  is

automatically  saved  to  "view87.par"  on  exiting  the program.  This

function saves the parameters without exiting.

-Auto  save-  when  ON  causes the parameter file to be updated after

every alteration.  It is still possible to crash this program,  and  it

is  less frustrating to have all the parameters saved before the crash.

So a setting of ON Is highly recommended for intial use.

-Orientation-  You  can select between Landscape (usual) and Portrait

(unusual but useful, especially if trying to produce Laser output). You

may want to select a higher resolution mode than 640x480  for  portrait

mode.

-Help line?  There is an optional line at the bottom  of  the  screen

which   will   display  brief  information  about  each  menu  command.

____________________________________________________________________________

GRAPH MENU

Graph-Display- from here,  a special set of functions  are  available

for manipulating the data on screen.  Briefly these are:

*******

Cursors:

A  number  of  operations  involve  the positioning of cursors.  For

example,  <Enter> selects the  cursors  for  fitting.  Further  <Enter>

keypresses  toggle  between  the  "Start" cursor and the "End" cursors.

Arrow keys and <Ctrl> move the cursors,  <Esc> fixes them and continues

with the program.

Most functions save the cursor values to unique memory locations,

even though the cursors look the same!

At the top of the screen,  you will see a brief reminder of what you

are doing,  and the cursor location described as X,Y either in whatever

units are chosen,  or as absolute  file  position  and  integer  value.

Pressing  "L"  or  "l" saves the current cursor position to a text file

(named as the Graph-Set cursors-Log file name).

Commands:

A : invokes the series resistance compensation, as set up in the

Transform-Compensate Rs menu.

C : clears the display and re-draws the trace

G : adds a grid to the display, independent of the scaling

I : invert the contents of the buffer being displayed

J  :  jump  forwards  or  backwards  in the data file by a number of

sweeps that you are prompted to enter,  or to the end "e" or  beginning

"b". To get an absolute sweep number press "s" <RETURN>, and then enter

the sweep number you wish to go to.

L  :  subtract  a baseline from the data.  First two cursors appear-

they indicate the section of  data  that  will  be  used  estimate  the

baseline.  You  can  move  these!   Press  <Esc>  to  continue.  Now  a

horizontal line appears,  with a box.  Either use arrows to improve the

baseline estimate,  or type in a number, <Esc> to continue and subtract

the baseline from the whole trace.

M :  get mean,  maximum and minimum between two  points.  First  two

cursors appear, to indicate the first region to be analysed. At the top

of the screen,  as usual, you see the coordinates of the cursors, and a

reminder of what you are doing.  You can move the cursors.  On pressing

<Esc>, the mean, minimum and maximum values are obtained and written to

a file (specified under the Graph-Mean/min/max- menu).  They  are  also

dislayed  on  screen.  You then move to the next section to be analysed

(until  all  sections,  set  in  Graph-Mean/min/max-  menu,  have  been

analyzed.

N :  integrate the data.  First position the two cursors to indicate

the section of data  to  be  integrated.  <Esc>  to  continue.  Next  a

baseline  estimate  is  made  by taking a mean between the two cursors,

which may be adjusted (see "L" above).  Lastly,  repeated  integrations

are  performed  using successively larger reduction factors,  until the

integraton may be performed without  an  out-of-range  error.  At  this

point,  the  integrated trace is displayed,  along with cursors to show

the  limits  of  integration,  and  a  modified  scale  (in  red).  The

integrated  trace  is  stored in the other buffer.  Press "x" to see it

and "x" to return to the data.

P :  toggle points/lines in the display.  Note that you will see  no

difference until the trace is re-drawn.

R  :  find transition point.  This shifts the trace in the buffer to

align  it  with  a  particular  data  feature.  See  the section Graph-

Average-Set alignment for a description.

S :  toggle scale lines across the whole screen  ON/OFF.  Note  that

when  you  turn the scale lines OFF,  they will not disappear until the

screen is redrawn.

t,T :  "t" tags the displayed trace (or untags it if it  is  already

tagged), then automatically loads the next trace.  "T" sets all tags to

be the same as the displayed trace's tag.  Lastly, <Alt>+"t" UNTAGS all

sweeps.  Tags are used in the Graph-Edit- menu for selecting sweeps  to

modify and save.

U : toggle update screen ON/OFF.  After pressing "U" the first time,

successive  screen  re-sizing  of the trace does not cause a screen re-

draw.  Pressing "U" a second time allows the screen to be re-drawn with

the next alteration.

W  :  change current window size.  The display windows are described

in detail in section 6.  Check the top R corner,  which should say  for

example,  "window  [0]"  to  indicate  that  you  are using the zero'th

window.  Pressing "W" allows the size to be altered.  First you  should

see  little  corners appear at the corners of the window.  These may be

moved using arrows, <Ctrl>, <Pg up> and <Pg dn>.  Press <Return> to fix

them and continue.  You can alter the size of any of the windows.

X :  exchange data buffer and  average  buffer.  The  label  average

appears  on  screen,  and  the  scale  in is red when using the average

buffer.  However the two  buffers  are  entirely  interchangeable,  and

would be better termed "displayed buffer" and "not-displayed buffer".

z : zero rotation on current trace (see Z, < and > below.

Z : zero rotation on all traces

+ : causes the current trace to be averaged with the contents of the

other (average) buffer.  The number averaged so far is displayed in the

top line as "buf [n]"

- :  invert the trace in the displayed buffer.

<Alt>+A    : is a short cut to the average sub-menu

<Alt>+Z :  clear the other (average buffer) and set the number averaged

to zero.

<spacebar> : next trace

<Tab>      : previous trace

Re-sizing and re-positioning the trace:

<,>  :  these  keys  shift (rotate) the data in the buffer one point

left or right.  <Shift> and "<" or ">" move the data by several points-

this movement is rapidly accelerated by multiple keystrokes.

<Insert>/<Delete : trace up/down

<Home>           : default size

<PageUp>/<PageDown> : enlarge, diminish X scale

arrows/<Ctrl>+arrows : move trace around screen, expand Y scale

<RETURN> : set two screen cursors for fitting a curve

Setting cursors:

<F1> : set a single screen cursor

<F2> : set two pairs of screen cursors for fitting. See the note at

the top for cursors in  general.

<F5>/<F6>/<F7> : swap display windows. <F5> to get window [0], <F7>

to go up, and <F6> to go down.  The total number of windows is set in

Graph-Number of windows:-.

<Esc> to get back to the Graph Menu.

<Alt>+"F" to go directly to the File menu,

<Alt>+"C" to go directly to the Copy menu,

etc.

<Alt>+ H to get help on the functions available from the Display

mode.

******

(BACK TO THE GRAPH- menu)

Graph-Set  cursors-  This  selects  the menu for manually setting the

screen cursors for fitting.

Graph-Mean,  min,  max- This selects the menu for recording the mean,

minumum maximum values, and variance of a section of data.

Graph-Average-  Select the Average menu for many functions associated

with averaging.

Graph-Integrate- A menu for various integration functions

Graph-Transient/Control-Transient   subtract  and  Control  subtract-

Menus for automatic subtraction of a single  scaled  transient  control

trace, or multiple controls from responses.

Graph-Edit file- Selects a menu from which  new  data  files  may  be

constructed, containing manupulated data from the current data file.

Graph-@event  detection-  Selects a menu for detecting events using a

threshold crossing technique.

Graph-H  window:- Graph-V window:- Graph-Number of windows.  There is

an option to set up a series of display windows.  If,  for example,  "V

window"  is  "4" and "H window" is "2",  then there will be available a

total  of 9 windows (window 0-8).  Window 0 is the largest window,  the

others  are arranged inside it in 2 columns of 4 windows each.

You may want to add additional windows,  so  simply  enter  a  number

larger  than  HxV  in "Number of windows".  When in the "GRAPH-DISPLAY"

mode,  you can swap between windows with F6 and F7.  You can also alter

the  size and shape of each window (including window [0]),  by pressing

"w" (as described above).

Note that F5 always returns you  to  window  0.  The  current  window

number should appear in the status line at the top R of the screen.

Graph-Cursor group- You may wish to retain particular settings of the

display magnification and the positions of the fitting  cursors.  These

are  saved  as "cursor groups".  Ten are available;  select the one you

wish to have current, the rest will remain unaltered.  The current

group is the one that is edited in the Set-Cursors- menu.

GRAPH-SET CURSORS- MENU

Graph-Set cursors -StartFit1 ..  -EndPlot- Set manually  the  cursors

for  selecting  the part of the data to fit,  and to display on screen.

StartFit1 and EndFit1 are the cursors  selected  by  pressing  <Return>

from the Display mode,  and are used in fitting.  StartFit2 and EndFit2

are selected by <F2> from the Display mode,  and are used in the "Split

run" mode of fitting under "Delp".  StartPlot is the point that appears

close to the left edge of the display window;  EndPlot appears  on  the

right edge.

Graph-Set cursors-Sweep number- Type in the number of  the  sweep  to

display on screen and to fit, filter, transform etc.

Graph-Set  cursors-Log file name- This is the name of a text or ASCII

file to which may be written the X and Y values of a displayed cursor.

Graph-Set  cursors-Reset log file-  Creates  an empty file with the

above file name.

GRAPH-MEAN, MIN, MAX- MENU

It is often useful to know the minimum, maximum, mean and variance of

the  points  lying  in  a  section  of data.  These may be obtained and

recorded from this menu.

Graph-Mean,  min,  max-Mean file name-  This is the name of the ASCII

text file to which the calculated values will be written.

-Get  mean,  min  & max-  First a pair of cursors appear on screen to

indicate the region of data over which to look for the values.  You can

move  these,  and  press  <Esc> to continue.  Next the values appear on

screen.  They are not written to a file.  If the "Number  of  sections"

is set greater than 1,  then you move on to the next section and repeat

the process of setting cursors and displaying the results.

-Overwrite  mean-  The  ASCII  string values from the last "Get mean,

min, max" operation replace the file specified above.

-Append  to  mean  file-  The ASCII string values are appended to the

file specified above.

-Number of sections- The number of sections of data  from  which  you

wish to find the mean, min etc.

-Start mean:, -End mean:  the range of data points (in points) may be

set manually from here.

-Output mode The mode for writing to the ASCII file. The options are;

Means only .. write only the mean values for each section

Mean and var .. write the mean and the variance only for each section

Mn,  var,  max ..  write the mean,  the variance and the minimum and

maximum for each section.

Info,  mean ..  like Means only,  except that  the  filename  and  the

beginning and end of each section are included in the output.

Info, mean, var .. like Mean and var, except ditto

All info ..  The whole caboodle:  filename,  beginnings and ends, mean

min, max, variance.

-Clear- Just clears the screen, not the ASCII file.

GRAPH-AVERAGE- MENU

(NB:  throughout,  the "average buffer" means whichever buffer is not

displayed on screen.)

Graph-Average-Clear buffer- Reset,  or clear the average buffer,  and

set the number of sweeps averaged to zero.

Graph-Average-File  name-  This  is the name of the file to which the

average buffer will be saved when using "Append" or "Overwrite"

Graph-Average-Overwrite file- The contents of the average  buffer  is

written  together with the current header (if any),  replacing any file

of the same name that exists.

Graph-Average-Append file- The contents  of  the  average  buffer  is

added/appended to the end of the file given above.

Graph-Average-Tag to data file- The contents of the average buffer is

added to the end of the data file. It is very bad practice to alter you

data  file  in this way,  so I do not know why it is still an available

option.

Graph-Average-"+" list- Type in a sequence of integers.  On selecting

"Run",   these  sweeps  of  data  wil   be   averaged.   For   example:

'1,2,3,5..10' means that sweeps 1,2 and 3 and  5  through  10  will  be

averaged.

Graph-Average-"-"  list  Type in the sweeps that should be subtracted

from the average.  On pressing "Run", these sweeps will be inverted and

then averaged along with the sweeps in the "+" list.

Graph-Average-Run- Make an average of all the sweeps in the  "+"  and

"-" lists.

Graph-Average-Set  alignment-  Automatically rotate all the sweeps in

the current file,  starting with and including the current sweeps using

some  feature  presumed  to  be  located between two cursors.  You will

first be prompted to reposition two cursors.

-With filtering- If this is set  to  "ON",  the  automatic  alignment

sequence  will  begin  by  filtering  the  section of data displayed on

screen, using the parameters in the "Filter" menu.

-Buffer  size-  The  automatic  rotation function works by taking the

Fourier transform of the data,  looking for a pattern in  the  relative

phase  of the first "Buffer size" Fourier components.  The more closely

the signal event in the data corresponds to a rectangular  event,  then

the larger the "Buffer size" can be.  In general a larger "Buffer size"

will give a more accurate result.  Somewhere between 2 and 6 is usually

suitable.  A  second phase has been added in which the halfway crossing

point in the identified transition is calculated, and the trace rotated

around this point.

If the buffer size is set to zero,  the Fourier alignment is  omitted

entirely, and the threshold crossing method used. But note that in this

case, you have to set two sets of cursors: the first is set to indicate

a section of data before the event,  the second to indicate the seciton

of data following the event.

-Mean fraction-  The  fractions  of  the  trace  that  are  used  for

calculating the threshold crossing point.  Set 0.3 for  most  purposes,

or 0.0 to omit this part of the procedure.

-Threshold (0 to 1)- A value of 0.5 means that the  program  searches

for  the point whose Y value is half-way between the Y values preceding

and following the edge event.

----------__-_-___----_-_-|

|_   <----- Threshold = 0.2

|

|   <-----  Threshold = 0.5

|

|

|

|-_----_--_---_-___---_--_-__-____---_--

GRAPH-INTEGRATE MENU

The way that the integration procedure works is as follows: a section

of data must be selected,  a baseline must be set,  a scale  factor  or

reduction  ratio must be used so that the integrated result does not go

out of range.

Graph-Integrate-Int. baseline- On selecting this, A horizontal cursor

appears on screen.  It is set initially  at  the  mean  of  the  points

between  A  and B (below).  It may be moved up and down using the arrow

keys,  or you can type in a value.  Whatever value is displayed in  the

box is subtracted from each data point before integrating.

Graph-Integrate-Reduction  1  to  -  The  current  reduction ratio is

displayed to the right.  It may  be  altered.  Note  that  the  display

scale  is  temporarily  adjusted  to  account  for this reduction.  Any

positive integer value may be entered (<32768).

Graph-Integrate-Single try-  Just  average  once,  with  the  current

settings.

Graph-Integrate-Multiple  tries- Repeat the averaging,  adjusting the

reduction ratio by a factor of two each  time.  When  the  trace  is  a

convenient  size then press the spacebar to stop,  otherwise reductions

continue until no out-of-range error is encountered.

Graph-Integrate-A:   start  int./-B:   end   int.   The   limits   of

integration, in data points.

Graph-Integrate-Mark  range-  When  this  is  selected,  two vertical

cursors appear on screen to mark the start and end of  the  integration

range.  They  may be moved with the arrow keys,  toggle between the two

with <Enter> and continue with <Esc>.

Graph-Integrate-Clear-  Clears  the display window,  and re-draws the

data.

GRAPH- .. -TRANSIENT SUBTRACT MENU

It is assumed that you have a set of data traces,  to which you  wish

to add a single trace, with systematic scaling factors:  for example, a

trace in response to a -20 mV step to be added to traces in response to

+40 mV, +50 mV, +60 mV steps etc.

First,  load  the  trace to be added in the "average" buffer by first

loading the file and entering <Alt+G> to get the display  mode.  REMOVE

ANY  CONSTANT OFFSET before the 20 mV step by pressing "L".  Next press

<Alt+Z> to clear the average buffer and then press "+" to add the trace

to the "average" buffer.  You now have the transient signal  ready  for

systematic  transient  subtraction.  Now  load  the  file with data (if

different) from the File menu, and go the Transient subtract menu.

-Subtracted  file  name  Enter the name of the file to which you wish

the results of the leak subtraction process to be saved.

-First ratio- Work out how much of the transient trace  you  wish  to

ADD  to the first data trace.  For example if the transient trace was a

response to a -20 mV step, and you wish to remove the transient part of

responses the first of which is to a +40 mV step, set tht "First ratio"

to - +40/-20 or +2.0.

-Increment- The ammount  of  the  transient  trace  to  be  added  is

incremented  by  this  fraction.  So  if  the  second  data  trace is a

response to a 50 mV step,  and the transient trace is a response to -20

mV, the "Increment" will be (50-40)/20 or 0.5.

-Number of trials- The number of data traces/sweeps in the file, that

you wish to have subtracted.

-Begin with sweep:- where in the data file to begin.

-Do  subtractions-  Begin  the  process of adding transient traces to

data traces.

Provided that multiple display windows exist,  you should see the raw

data  and  the results of transient/leak subtraction appear in separate

windows.

-Mode- There is a choice of subtraction modes:  Transient only,  Leak

only and Leak and Transient.

"TRANSIENT" only causes the scaled contents of the average buffer to

be added to the curent trace before saving.

"LEAK  ONLY" and "leak&transient" removes a constant offset from the

each data trace,  and "leak&transient"then goes on to  add  the  scaled

transient trace in the average buffer.

If using a "leak" mode, then on loading the first data trace, cursors

will  appear  so  that  you  can  select  the  region  of data to set a

baseline.   Press  <Esc>  and  re-adjust  the  baseline   estimate   if

necessary.  Press  <Esc>  again,  and  that baseline will be subtracted

from the data.  As successive traces are loaded,  you are only required

to confirm the baseline estimate and press <Esc> once.

-Reset  subtract  file-  This function creates an empty file with the

file name specified above. Existing data will be lost!

NB remember to CHECK the modified data to make sure that you have  done

the  subtractions  correctly.  It  is  easy to make a mistake in "First

ratio" and "Increment".

GRAPH- .. -CONTROL SUBTRACT- MENU

Essentially this feature allows you to select a file containing a set

of traces that you wish to subtract systematically from the set of data

traces in the file loaded from the "File" menu.  The process may  begin

at  any  trace number in either file.  The results are saved to a third

file.  Note that all three files may  be  the  same  if  required  (not

recommended  on  principle).  Note  also  that if the files do not have

identical digitization frequencies and numbers  of  points  per  sweep,

then you will get rubbish.

-Response  file  name-  To  this file will be appended the results of

subtracting the control file traces from the data file traces.

-File to subtract- The file containing the controls to be subtracted.

Note that this may be the same file as the data.

-Data start:- Start at this sweep/trace in the data.

-Control start:- Start at this sweep/trace in  the  control  file  to

subtract.

-Number of controls:- The number of consecutive subtractions you wish

to make.

-Multiply  control  by:-  So someone is bound to have collected their

data  and controls at different gain!  DON'T do this!  However,  if you

have,  then all is not lost.  You will need to change  the  control  to

match   the  data,   so  enter  the  correct  re-scaling  factor  here.

Subtracted responses will be saved with the same scaling  as  the  data

from the "File to subtract".

-Subtract controls- Begin the process of subtraction.

-Reset  response file- The results of the subtraction process will be

appended to the "Response file" so use this to clear pre-existing  data

in the response file.

GRAPH-EDIT DATA FILE MENU

Using  this  menu,  it is possible to construct a new file containing

one  or more sweeps from the current data file,  modified by filtering,

inversion, leak or baseline subtraction, integration or multiplication.

-Edited-file  name:  Enter  the name of the file to which you wish to

append the modified data.  Entering "=" <Enter> gets you the  path  and

name  (no  extension) of the current data file,  which may be edited by

starting with a backspace.

-A:  write  tagged swps.  -This goes through the process of modifying

the sweeps that you tagged from the Display-Graph screen,  and  writing

them sequentially to a file.

-B:  write  un-tagged swps.  Identical to A:,  except that the sweeps

that are not tagged are used.

-C:  write listed sweeps.  -This does the same as A:  except that  it

parses  the  list of sweeps that can be entered with the function "Make

output list" below.

-D:  write this  buffer.  -Operates  on  only  the  contents  of  the

displayed buffer.

Make output list- Enter a list of sweeps numbers punctuated by commas

and stops eg 1,2,3..10 means all sweeps 1 to 10 inclusive.

Filter  sweeps- filter using the current settings,  before writing to

the file.

Leak subtract- with the  first  sweep,  you  are  prompted  to  place

cursors  around  a  section  of data to used to calculate the offset or

leak.

Invert each sweep- should be self-explanatory.

Integrate each sweep- A single integration is  performed,  using  the

limits,  baseline  and  reduction  factor  specified in Graph-Integrate

menu-.

X multiply each sweep- by the value entered  under  Tranform-Multiply

by:-

Output mode:- either two byte  integer (normal data format) or ASCII

X,Y real format.

Save marked segment- instead of saving a whole sweep,  you  can  save

the  section  of  data between two cursors that you will be prompted to

re-position.  Use this with discretion!

Reset edit-file.  This function creates a file containing zero bytes.

@EVENT DETECTION MENU

This is a threshold crossing routine.  An event is considered to have

started  when  the  Y  value crosses "Threshold value ON",  and to have

ended when it crosses "Threshold value OFF".  The crossings have to  be

in the right direction ("Direction to look").

Detection begins at any point in a data file,  specified either as  a

number  of sweeps into the file,  or as an absolute time from the start

of the file.

Data are read in consecutive blocks of "Blocksize".  When an event is

detected, it may be saved/snipped to a new file.

Note that if a differentiating filter is selected from the Transform-

Filter-Type  menu  (Dsinc,  3ptDiff)  and "Filter first",  this routine

becomes a gradient sensitive detection routine,  useful for example  in

detecting synaptic events.

-Offset time- the absolute time, from the beginning of the file, when

to start detection.

-Num  of  swps  to offset- start at a number of sweeps (of the length

specified in the Tranform-Rescale menu) from the beginning of the file.

-Blocksize- the size of the blocks of  data  that  are  read  in  and

displayed  in  the process detecting events.  This has no effect on the

way detection works,  only on what is shown to the  user.  However,  if

this  value  is  set too large,  a block of data at the end of the data

file may not get analysed if it is less than "Blocksize".

-Threshold value ON:  this will display a horizontal cursor on screen

which may be repositioned. You can also type in the value.  An event is

considered to have started if the data crosses this threshold.

-Threshold value OFF: ditto

-Direction  to  look;  If  you  wish to detect events that occur in a

downwards direction from baseline, then set this to DOWN

-Filter first: Set to ON, and each block will be filtered on loading.

The output or saved/snipped sections will not be filtered.

-Points to snip:  the total number of points to save when an event is

detected.

-Lead length:  save from this many points before the detected  event.

So  the  total  number of points saved for each detected event is "lead

length" plus "Points to snip". Don't stint on this value!

-Remove offset:  before  saving,  subtract  from  the  whole  snipped

segment  the  mean  value  of the data contained in the lead segment of

data.

-Snip file name:  the name of the file where snipped sections will be

saved.

-Reset snip file: create an empty file with the name above.

-Interrupt?- When ON,  you will be  prompted  before  each  event  is

saved.

-begin detection- as it says, begin event detection.

___________________________________________________________________________

COPY MENU

The   organization   of  this  menu  may  seem  a  little  confusing.

Essentially there are three components:  Dot matrix output, Output mode

and destination,  Multiple plottiny utilities for creating plots of all

sweeps in file for example.

Copy-Dot matrix dump- This puts out  a  pixel-by-pixel  plot  of  the

current screen display. 1024x768 displays are supported.  It works on a

Epsom  LX-80  and LQ-800.  It should work with others as well,  but you

may have  to  fiddle  with  Dip  switches.  This  is  the  only  output

available for plain dot matrix printers.

Copy-Laser  menu-  High  resolution  output  is  available on a laser

printer, with control over the final format: landscape mode only at the

moment is available.  The fine control over print modes,  are contained

in this menu and sub-menu.

Copy-Output:-  The  destination  for the printer output.  For example

"PRN",  "LPT1",  "graph.tmp",  "plotfile.tmp"  etc.  (quote  marks  not

required).

Copy-DOS print- Provided you have enough program memory available,  a

DOS shell is invoked,  and the current  output  file  is  sent  to  the

printer.  The advantage to plotting to a file first and the printing is

that  you  can  get  your  plot  file right before printing it.  If you

decide to leave the program before  printing  your  output  file,  then

don't  forget  to  use the batch command "NVPRINT filename" to send the

plot file to the printer.

Copy-Reset output:- If this function is used,  any data saved in  the

current  destination  will  be  deleted,  and  replaced  with a "Reset"

command.  So if you have several plto  files  that  you  wish  to  have

printed on the same page, then do not use this command. Instead, delete

the  files  from  the  DOS  command  prompt  before printing to them in

VIEWMENU.

Copy-Plot all sweeps- All the sweeps  in  the  loaded  file  will  be

displayed on screen, and plotted to a laser printer or file only, using

the  settings  in  the  laser menu.  NB unless you use one of the "fast

plot" options in the Laser menu,  then this could a) take a  very  long

time b) cause a memory error in the printer.  Note that the settings of

"Auto windows" and "Filter plots" below affect the plot output.

Copy-Contents  of  dir-  This  is  essentially  the  same as Plot all

sweeps, except that the extension entered is used to search the current

directory (selectable from the File-Change dir  command)  and  all  the

files  found  are plotted:  a sample trace form the middle of the file,

and the last trace (with my data the last trace is the average  of  the

preceding  traces).  Note  that  the  settings  of  "Auto  windows" and

"Filter plots" affect the plot output.

Copy-Extension- This is the file extension used to search for files

to plot.

Copy-Filter plots- When this option is ON, then all plots made using

"Plot all sweeps" or "Contents of dir" are first filtered with whatever

filter settings you last used.

Copy-Auto  windows- There are three settings to this function:  AUTO,

PRESET and ONE WINDOW.  How it is set,  determines how the "Contents of

dir" and "Plot all sweeps" functions operate.

In the AUTO mode, 16 windows are created, and a trace put into each.

In  the PRE-SET mode,  the windows created from H,  V and N under the

GRAPH menu are used, and a trace put in each.

In the ONE WINDOW mode,  the currently selected window is  used,  and

all the traces put in this window.

Note  that the positions at which that the scale bar and the text are

placed depends on this setting,  and on whether sweeps in  a  file,  or

directory  contents  are  being  plotted.  The following truth table is

applicable:

PLOT ALL SWEEPS

SCALE BAR                    TEXT POSITION

PRESET      absolute                      relative

AUTO        absolute                      calculated

ONE WINDOW  relative                      relative

CONTENTS OF DIR

PRESET      relative                      relative

AUTO        calculated                    calculated

ONE WINDOW  absolute                      absolute

where "absolute" means that the item is placed at the  position  that

you  set  from  the  LASER  MENU  option,  "calculated"  means that the

position to put the item is calculated  automatically,  and  "relative"

means  that  you should set the position in the best place in the first

window (window [1]) and in subsequent windows,  the item will appear in

a similar position.

Copy-Draw  boxes-  If  this  is  ON,  then a box is drawn around each

window when the trace in the  window  is  sent  to  the  printer.  This

effect is global.

Copy-Mode  (HPGL/LJ+)- Different command sets are written to the plot

file depending on the mode selected.  The HPGL mode  uses  the  minimum

model  settings,  LJ+ mode uses the HPII PCL instruction set.  Swapping

print modes causes the plot destination to  be  reset  (see  notes  for

"Reset output" above). Note that to print out the plot in LJ+ mode, the

special fonts in the file "hpfont1" must be loaded to the printer.  Use

the /b option when copying to the printer.  Alternatively use the batch

file "nvprint.bat" to print LJ+ files.

Copy-Image clipping- This can be either ON or OFF. It only has effect

on the plotted ooutput if Draw boxes is Off. Essentially, any trace can

be extended outside of its window if "Image clipping" is Off. If "Image

clipping" is ON,  parts of a trace that go over the edge of a window do

not appear in the printer output.

This function also affects what appears on screen,  independently  of

"Draw boxes".

COPY-LASER MENU

-Plot  style-  This  function  toggles  between  plotting  individual

points,  or drawing lines (continuous or dashed)  between  the  points.

Note  that  two  fast plotting options are available:  "Fast lines" and

"Fast dots".  These can be vastly faster (especially  "Fast  dots")  as

not  every  point  is plotted.  Increasing the dot size with these fast

plotting styles further increases the plotting speed.

-Dot size- Whatever symbol is selected,  printed in  a  size  ranging

from 1 (almost invisible) to 16 (about 4 mm across) or 32 in HPGL mode.

-Symbol-  Toggle between a range of symbols.  At the moment,  circles

and squares are available for PCL, and all for HPGL mode.

-Laser plot- When this function is selected,  the current data  sweep

as  displayed  on  screen is output to the printer.  At the same time a

scale bar may also be printed,  along with some information such as the

location  of  the cursors marking the fitted region,  the file name and

sweep number,  and the results of the last DELP fit (depending on  what

further options are set: see below).

-Bar scale- Toggle On/Off;  when ON a scale bar will be printed along

with  the  data.  When this option is changed from OFF to ON,  a cursor

should appear on the screen, which may be moved with the arrow keys. If

its location is not obvious,  then press <Home> to put it in the middle

of  the  screen.  Press  <Return> to fix the position and return to the

menu.

-Text start point- All text printed  out,  the  file  name,  the  fit

results etc will be printed out started from the "Text start point".  A

cursor  will  appear  on  screen on selecting this function.  It may be

moved using the arrow keys.  Press <Return> to  fix  the  position  and

return to the menu.

-Write  over-  When  this  function is selected,  a window appears on

screen showing the last message  that  was  entered  here.  It  may  be

edited.  On pressing <Return>,  the message will be written immediately

to the plot  destination,  and  also  to  the  screen  in  roughly  the

comparable position and size.

-Eject  page-  This function sends to the plot destination the signal

to dump out a sheet.

-Fitting info.- This toggles ON/OFF.  When "ON",  the results of  the

last  DELP  fit will be sent to the printer when the next data trace is

printed using "Laser plot".  It will appear  below  the  point  set  by

"Text start point".

-Change  font-  Some  control  over  the  fonts  used  in printing is

available from this menu.  The  selected  fonts  differ  in  the  laser

output only, not on the screen.

-Number  of copies- Select how many copies of the completed output to

be dumped on selecting "Eject page".

-Indicate fit window- Toggles between "Lines", "Bar" and "Off".  When

"Lines",   vertical  dashed  lines  appear  in   the   plotted   output

corresponding  to  the  region  of data used in the fitting procedures.

When first set to "Bar",  a target appears on the screen which  may  be

moved around the screen:  when a trace is plotted,  a bar indicates the

region of data  used  for  fitting,  and  is  placed  at  the  vertical

coordinate of the target.

-Zero-scale  line-  When  this  option is on,  a thin horizontal line

appears  on  the  plotted  output  at  a   position   indicating   zero

(current/voltage etc).

-Reset plot default- All the parameters in the  Laser  menu  will  be

returned  to  default values.  This feature is useful in constructing a

macro to plot with particular plotting features selected.

COPY-LASER MENU-CHANGE FONT MENU

-A:  scale bar size- An integer value given here will select the size

of  the  text associated with the scale bar.  Note that a corresponding

font size must be available on the printer.  This  is  most  useful  in

conjunction with PCL and proportional fonts.

-B: scale bar font- Toggle between the various fonts supported (those

found on a standard HP II, III and 4 printer).

-C: scale bar weight- If this available on your printer, then you can

select various weights of text to be used with the scale bar.

-D:  scale  bar Italic- Makes the text asssociated with the scale bar

Italic.

-E:, F:, G:, H:  text size, font, weight and Italic- These relate to

all other text output to the  printer:  the  fitted  output,  the  text

entered in "Write over" and the file information printed automatically.

-Proportional  spacing-  To  get  the  full  range  of  font   sizes,

proportional spacing should be ON.

-Reset default- These fonts are reset to those that work with a HP II

printer.

#